TYCHE R2-T1

04_Clean_Aero

G1.4–G1.6 Mixed-Model Geometry-Repair Analysis Log

OpenVSP / VSPAERO 3.51.2  |  Analysis closure record  |  20 August 2026

Purpose: preserve the complete G1.4–G1.6 diagnostic trail before hand-off to 05_Stability, including geometry changes, numerical evidence, physical-validity gates, dispositions, and the final 04_Clean_Aero folder conclusion.

# 1. Scope and controlled analysis basis

The G1.4–G1.6 campaign was opened after the original mixed thin/thick G2 clean-aerodynamic model generated nonphysical root slivers, malformed effective chords, large sectional-load spikes, surface/Trefftz disagreement, and zero-sideslip asymmetry. The objective was not to tune VSPAERO to recover a preferred report value. The objective was to determine whether a physically credible mixed thin-lifting/thick-body representation could be established for Tyche R2-T1 and, if not, to bound and document the model-form limitation.

## 1.1 Reference and solver controls

| Control | Value |
| --- | --- |
| Reference area, Sref | 30.0 m² |
| Reference span, Bref | 16.4 m |
| Reference chord, Cref | 1.881 m |
| Moment reference | Xref = 3.750 m; Yref = 0; Zref = 0 |
| Diagnostic flow condition | α = 4°, β = 0°, M = 0.203, ReCref = 6.98×10⁶ |
| Wake / iteration controls | 16 wake nodes; 20 iterations; relaxation 1.0; fixed wake OFF |
| Production mesh state | MainWing W73, U = 9/37/7/19; HTail W73/U29; VTail W73/U29 |
| Symmetry | XZ symmetry OFF; physical left/right symmetry assessed from loads and coefficients |

Configuration-control note: several G1.5/G1.6 scripts retained lower tessellation defaults, but the executed diagnostic .vsp3 cases were manually restored to the established Extra-Fine values before VSPAERO. The CSV results, not the script defaults, are the authoritative solver-state evidence.

## 1.2 Acceptance philosophy

A run could converge numerically and still fail physically. Therefore coefficient flatness was never used as the sole acceptance criterion. A configuration was retained only when geometry topology, iterative convergence, symmetry and surface/wake sanity were jointly credible.

| Gate | Working criterion |
| --- | --- |
| Final-step lift | ¦ΔCL¦ < 0.5% |
| Final-step wake lift | ¦ΔCL,w¦ < 0.5% |
| Final-step Trefftz induced drag | ¦ΔCDi,w¦ < 0.5% |
| Final-step pitching moment | ¦ΔCMy¦ < 0.005 |
| Zero-sideslip symmetry | ¦CS¦ and ¦Cn¦ ≲ 10⁻³ for isolated symmetric diagnostics |
| Root topology | No clipped ~0.02–0.10 m remnants, no multi-metre malformed chord, mirrored left/right geometry |
| Residual quality | Prefer log10(L2) < −2 without late growth; interpret together with physical checks |
| Drag interpretation | Trefftz CDi,w primary for mixed-model diagnosis; PDT remains source of parasite drag |

# 2. G1.4 analysis log — MainWing root repair and first H2 failure

## 2.1 G1.4-H1 Mode 1 — blended WingRootFairing

The first WingRootFairing repair removed the original catastrophic wing/body root topology. The first exposed MainWing station became physically plausible (approximately y = ±0.629 m, c = 2.279 m), replacing the earlier near-zero / oversized root-strip behaviour. A localized first-strip load spike remained close to the fairing edge.

| Result | Value | Assessment |
| --- | --- | --- |
| CL / CLw | 0.670999 / 0.647364 | Global lift stable |
| CDi / CDi,w | 0.015032 / 0.017530 | Surface CDi noisy; wake value usable diagnostically |
| CMy | −0.008925 | Moment numerically bounded |
| CS / Cn | +7.93×10⁻⁵ / +4.64×10⁻⁵ | Excellent global symmetry |
| Root load | cl ≈ 3.47 at first strip | Localized edge artifact remains |
| Disposition | CONDITIONAL | Topology fixed; local fairing-edge sensitivity remains |

Source: TRIAL.csv; G1.4 script.

## 2.2 G1.4-H1 Mode 2 — enclosed WingRootFairing

FAIRING_MODE = 2 moved the first exposed wing station farther from the body and reduced the first-strip singularity. The physical root chord was retained and global symmetry remained good. This became the working AC-02 wing/body repair.

| Result | Value | Assessment |
| --- | --- | --- |
| First exposed root | y ≈ ±0.6967 m; c ≈ 2.2705 m | Physical planform chord |
| CL / CLw | 0.689660 / 0.645805 | Stable global lift |
| CDi / CDi,w | 0.010675 / 0.018459 | Near-field drag remains model-sensitive |
| CMy | −0.019213 | Final-step ¦ΔCMy¦ = 0.00320; pass |
| CS / Cn | −5.50×10⁻⁴ / −4.08×10⁻⁵ | Acceptable symmetry |
| Disposition | RETAIN | AC-02 working repair; do not retune wing geometry |

Source: H1_MODE2_MW_FUSE_FAIRING_A4.csv.

## 2.3 G1.4-H2 — add HTail and VTail

Adding the tails exposed a separate empennage intersection failure. The MainWing repair survived, but the HTail root collapsed to a ~0.103 m remnant followed by a ~1.679 m strip, with enormous local coefficients. The global solution was physically invalid.

| Indicator | Observed | Disposition |
| --- | --- | --- |
| HTail root topology | c ≈ 0.103 m → 1.679 m | FAIL |
| Representative HTail spike | cl ≈ 144–150; cdi ≈ −1.4×10⁴; cm ≈ −6.6×10⁴ | FAIL |
| CL / CLw | 0.391234 / 0.610084 | Severe disagreement |
| CDi / CDi,w | −0.014973 / +0.016970 | Nonphysical near-field drag |
| CMy | +0.852438 | Contaminated |
| CS / Cn | +0.017661 / +0.006049 | Zero-β symmetry fail |
| Disposition | REJECT | Localised to HTail/body root topology |

Source: H2_Mode_1.csv.

# 3. G1.5 analysis log — TailRootFairing and direct tail-cone penetration

## 3.1 G1.5-H2 with dedicated TailRootFairing

A dedicated TailRootFairing removed the gross 0.103 m HTail chord remnant in the combined H2 geometry, but introduced a strongly asymmetric first-strip intersection state. The geometry looked better while the pressure solution remained invalid.

| Indicator | Observed | Assessment |
| --- | --- | --- |
| HTail chord | First exposed chord ≈ 1.654 m | Gross sliver removed |
| Local asymmetry | One side cl ≈ −46; opposite side ≈ −0.58 | Severe one-sided singularity |
| CL / CLw | 0.599320 / 0.626331 | Moderate global mismatch |
| CDi / CDi,w | −0.018803 / +0.018100 | Near-field sign failure |
| CMy | +0.262228 | Contaminated |
| CS / Cn | +0.004594 / +0.001236 | Physical symmetry fail |
| Disposition | REJECT | TailRootFairing side-wall exit topology unresolved |

Source: H2_Mode_1(1).csv; G1.5 script.

## 3.2 Isolated HT_TEST with TailRootFairing

The isolated HTail + thick fuselage/TailRootFairing test converged exceptionally well but remained physically wrong. This was an important demonstration that residual convergence alone cannot validate a mixed-model result.

| Result | Value | Assessment |
| --- | --- | --- |
| CL / CLw | −0.030232 / +0.044663 | Opposite-sign lift evidence |
| CDi / CDi,w | −0.032355 / +0.000760 | Near-field result nonphysical |
| CMy | +0.095085 | Contaminated by root singularity |
| CS / Cn | −0.007400 / −0.003405 | Strong asymmetry |
| Residuals | log10(L2)=−3.841; log10(Rmax)=−2.303 | Numerically excellent |
| Disposition | REJECT | Converged-to-invalid solution |

Source: HT_TEST.csv.

## 3.3 Direct tail-cone penetration: fairing OFF, TAIL_CONE_RISE = 0.060

The TailRootFairing was removed and the tail-cone crown raised by 60 mm to create direct, symmetric penetration. This isolated test was the first convincing HTail/body success: lateral coefficients collapsed toward zero, global coefficients became plausible, and residuals converged strongly.

| Result | Value | Assessment |
| --- | --- | --- |
| CL / CLw | 0.045955 / 0.046670 | Good agreement |
| CDi / CDi,w | 0.001298 / 0.000695 | Surface/wake difference remains, but both positive |
| CMy | −0.138492 | Physically plausible tail moment |
| CS / Cn | −1.38×10⁻⁴ / +6.35×10⁻⁵ | PASS symmetry |
| Residuals | log10(L2)=−3.392; log10(Rmax)=−1.606 | PASS |
| Disposition | PASS DIAGNOSTIC | Direct HTail penetration viable in isolation |

Source: HT_Test_2.csv.

## 3.4 Combined H2 and H2_NO_VT with 60 mm rise

The isolated HTail repair did not survive full combination. With the MainWing/WRF system added, the complete H2 solution again became asymmetric and non-convergent; removing the VTail did not cure the failure. This ruled out the VTail as a necessary trigger and established a coupled multi-component problem.

| Test | CL / CLw | CDi / CDi,w | CMy | CS / Cn | Residuals | Disposition |
| --- | --- | --- | --- | --- | --- | --- |
| H2_Test_2 | 0.6579 / 0.6202 | 0.0120 / 0.0182 | +0.0750 | +0.00923 / +0.00294 | L2 +0.784; Rmax +2.869 | FAIL |
| H2_NO_VT_TEST | 0.5413 / 0.6124 | −0.00405 / 0.01717 | +0.4256 | −0.00267 / −0.00051 | L2 +0.714; Rmax +2.618 | FAIL |

Sources: H2_Test_2.csv; H2_NO_VT_TEST.csv.

# 4. G1.6 analysis log — predictive trim-boundary repair and campaign closure

G1.6 replaced the earlier tail-fairing logic with an explicit camber-plane trim-boundary strategy. The goal was to force a constant HTail inner boundary and eliminate narrow or malformed residual strips before solver time was committed.

## 4.1 Initial G1.6 TailRootFairing: 0.600 m width

The new fairing established a constant trim boundary at y = ±0.300 m and restored the exact local HTail planform chord. The old sliver mechanism was closed, but the first surviving strip was only ~28% of the normal spanwise width.

| Test | WRF in HT thick set? | CL / CLw | CDi / CDi,w | CS / Cn | Residuals | Disposition |
| --- | --- | --- | --- | --- | --- | --- |
| HT_TEST_1 | YES | 0.10584 / 0.03949 | 0.01368 / 0.000659 | +0.07662 / −0.00349 | L2 −1.072; Rmax +0.366 | HOLD/FAIL |
| HT_TEST_2 | NO | 0.04489 / 0.04461 | −0.00485 / 0.000818 | +0.00142 / +0.000616 | L2 −0.721; Rmax +0.698 | HOLD |

The one-flag A/B comparison showed that removing WingRootFairing eliminated most of the gross pressure asymmetry, but did not restore residual convergence. The remaining 28%-width first strip therefore remained a credible numerical sensitivity.

## 4.2 Final AC-04 fairing geometry: 0.680 m width

The fairing was widened and shifted forward-to-full transition aft: x_nose = 8.400 m, x_full,fwd = 8.600 m, width = 0.680 m (half-width 0.340 m). The first surviving HTail strip became 0.10176 m wide, about 92.1% of a normal strip, while the first-strip chord remained exactly consistent with the analytical tail planform.

| Test | Configuration | Key result | Disposition |
| --- | --- | --- | --- |
| HT_TEST_3 | HTail thin; Fuselage+TRF thick; WRF OFF | yinner = 0.34000 m; c = 1.63110 m; CL=0.045154; CDiw=0.000796; CMy=−0.131935; CS=−3.58×10⁻⁴; L2=−3.696; Rmax=−2.236 | PASS DIAGNOSTIC |
| HT_TEST_4 | Same improved TRF; WRF ON | Geometry and residuals remain excellent, but CS=+0.076276; CL/CLw=0.108647/0.042133; surface CDi=0.018347 vs CDiw=0.000663 | NUMERIC PASS / PHYSICAL FAIL |

AC-04 was therefore closed as a geometry-topology repair. The remaining failure was no longer a malformed HTail root: it was a global thick-body/multi-body pressure-field problem.

## 4.3 Combined repaired H2_NO_VT

The repaired MainWing and repaired HTail were combined with Fuselage + WingRootFairing + TailRootFairing. Both root geometries remained physically correct, but the global model again failed zero-sideslip symmetry and convergence.

| Metric | Result | Assessment |
| --- | --- | --- |
| MainWing root | y ≈ ±0.6973 m; c ≈ 2.2703 m | AC-02 retained |
| HTail root | y ≈ ±0.3909 m; c = 1.63110 m; inner edge 0.34000 m | AC-04 retained |
| CL / CLw | 0.697529 / 0.627483 | 11% scale disagreement |
| CDi / CDi,w | 0.020357 / 0.018027 | Final-step CDiw variation 3.97% |
| CMy | +0.073604 | Not reliable |
| CS / Cn | +0.050463 / −0.001902 | Severe zero-β physical failure |
| Residuals | log10(L2)=−0.284; log10(Rmax)=+1.349 | FAIL |
| Disposition | REJECT | Combined repaired mixed model not robust |

Source: H2_NO_VT_TEST_1.csv and archived H2_NO_VT .vsp3.

## 4.4 Final forensic H2: long TailRootFairing aft closure

A final analysis-only causality test extended the TailRootFairing aft point from FS 10.940 m to FS 11.500 m, lengthening the closure taper from 0.150 m to 0.710 m. This reduced the local aft-closure pressure peak but did not recover a valid global mixed-model solution.

| Metric | Result | Interpretation |
| --- | --- | --- |
| Local TRF aft-closure Cp | Reduced from approximately +0.83–0.93 to ≈ +0.325 | Abrupt short closure confirmed as local defect |
| CL / CLw | 0.703178 / 0.624349 | Large disagreement remains |
| CDi / CDi,w | 0.016150 / 0.018344 | Final-step CDi and CDiw fail convergence |
| CMy | +0.054457 | Final-step ¦ΔCMy¦ ≈ 0.00727; fail |
| CS / Cn | +0.054412 / −5.8×10⁻⁵ | Gross side force persists |
| Residuals | log10(L2)=−0.278; log10(Rmax)=+1.215 | FAIL |
| Lifting-surface roots | MainWing, HTail and VTail regular | Local root topology no longer dominant |
| Disposition | REJECT / CAMPAIGN CLOSED | Multi-body mixed thick/thin model-form limitation |

Source: H2_TEST_1.csv; Tyche_R2T1_G1_6_Analysis_H2.vspscript.

# 5. Consolidated G1.4–G1.6 result ledger

## 5.1 G1.4–G1.5 results

| Test | Primary CSV | CL | CDi,w | CMy | CS | log10 L2 | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G1.4-H1-M1 | TRIAL | 0.6710 | 0.01753 | −0.0089 | +0.00008 | −0.145 | Conditional |
| G1.4-H1-M2 | H1_MODE2 | 0.6897 | 0.01846 | −0.0192 | −0.00055 | +0.265 | Retain AC-02 |
| G1.4-H2 | H2_Mode_1 | 0.3912 | 0.01697 | +0.8524 | +0.01766 | +0.167 | Fail |
| G1.5-H2-TRF | H2_Mode_1(1) | 0.5993 | 0.01810 | +0.2622 | +0.00459 | −0.295 | Fail |
| G1.5-HT-TRF | HT_TEST | −0.0302 | 0.000760 | +0.0951 | −0.00740 | −3.841 | Fail physical |
| G1.5-HT-RISE060 | HT_Test_2 | 0.0460 | 0.000695 | −0.1385 | −0.00014 | −3.392 | Pass diagnostic |
| G1.5-H2-RISE060 | H2_Test_2 | 0.6579 | 0.01822 | +0.0750 | +0.00923 | +0.784 | Fail |
| G1.5-H2-NOVT | H2_NO_VT_TEST | 0.5413 | 0.01717 | +0.4256 | −0.00267 | +0.714 | Fail |

## 5.2 G1.6 results

| Test | Primary CSV | CL | CDi,w | CMy | CS | log10 L2 | Disposition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G1.6-HT1 | HT_TEST_1 | 0.1058 | 0.000659 | −0.0756 | +0.07662 | −1.072 | Hold/Fail |
| G1.6-HT2 | HT_TEST_2(1) | 0.0449 | 0.000818 | −0.1310 | +0.00142 | −0.721 | Hold |
| G1.6-HT3 | HT_TEST_3 | 0.0452 | 0.000796 | −0.1319 | −0.00036 | −3.696 | Pass diagnostic |
| G1.6-HT4 | HT_TEST_4 | 0.1086 | 0.000663 | −0.0789 | +0.07628 | −3.917 | Fail physical |
| G1.6-H2-NOVT | H2_NO_VT_TEST_1 | 0.6975 | 0.01803 | +0.0736 | +0.05046 | −0.284 | Fail |
| G1.6-H2-LONG | H2_TEST_1 | 0.7032 | 0.01834 | +0.0545 | +0.05441 | −0.278 | Close campaign |

# 6. Engineering conclusions from the geometry-repair campaign

AC-02 (MainWing root) is closed. WingRootFairing Mode 2 replaced the original clipped/oversized root topology with a physical first exposed chord of approximately 2.270 m.

AC-04 (HTail root) is closed. The final 0.680 m TailRootFairing generated a constant camber-plane trim boundary at y = ±0.3400 m and a first-strip chord of 1.63110 m matching the analytical planform.

Mesh-density insufficiency is not the controlling explanation. The decisive G1.5/G1.6 diagnostic cases were executed at the established Extra-Fine surface discretisation.

Numerical convergence is not sufficient evidence of physical validity. HT_TEST and HT_TEST_4 converged strongly while producing nonphysical lift/drag or zero-sideslip side force.

The abrupt 0.150 m TailRootFairing aft closure was a real local pressure defect, but not the dominant global failure mechanism. Lengthening the taper to 0.710 m reduced the local Cp peak without eliminating the spurious CS.

After all three lifting-surface root topologies became physically regular, the full mixed thin/thick assembly still showed large zero-β side force, poor residual convergence, and surface/wake inconsistency. The remaining error is therefore classified as a multi-body mixed-model / pressure-integration model-form limitation.

No H3 run is justified. Further fairing dimensions, higher tessellation, more wake nodes or more iterations would have low expected validation value relative to the remaining competition schedule.

# 7. 04_Clean_Aero folder conclusion

The 04_Clean_Aero campaign is CLOSED for the purpose of the Tyche R2-T1 competition analysis. The production clean-aerodynamic model is the converged thin lifting-system branch (MainWing + HTail + VTail + lift struts, with propulsion off), while the G1.4–G1.6 mixed thin/thick branch is retained only as validation and model-form sensitivity evidence.

## 7.1 Production aerodynamic disposition

| Element | Final finding | Use |
| --- | --- | --- |
| CA-G0 clean lifting | Baseline thin lifting surfaces | Reference polar / no-strut baseline |
| CA-G1 strut sensitivity | Struts added; small global effect; production lifting set selected | CLOSED / retained |
| CA-G3 repeatability | α=4°: CL=0.825049, CDi=0.030266, CMy=+0.002280; CLw=0.826879, CDi,w=0.022618 | Repeatability PASS |
| G2/G1.4–G1.6 mixed body | Multiple root defects repaired, but combined multi-body solution remains physically invalid | REJECT for production coefficients |
| Parasite drag | OpenVSP Parasite Drag Tool / controlled G2 drag build-up | Use PDT CD0, not VSPAERO CDo |
| Final drag build-up | CD = CD0,PDT + CDi,VSPAERO | Apply clean-lifting induced drag with documented mesh uncertainty |

## 7.2 Ready-to-paste folder closure statement

04_Clean_Aero was closed after completing the clean lifting-surface polar, strut sensitivity, repeatability checks, and a bounded mixed thin/thick geometry-repair campaign. The converged thin lifting-system model is retained as the production aerodynamic source. MainWing and HTail root-intersection defects identified in the mixed-body branch were independently repaired and verified at Extra-Fine discretisation; however, the fully combined thick-body model continued to produce nonphysical zero-sideslip lateral force, residual oscillation and surface/Trefftz inconsistencies even after the local root topologies and the TailRootFairing aft closure were corrected. The mixed-body solution was therefore rejected under the predefined physical-validity gates and retained only as model-form sensitivity evidence. Parasite drag remains sourced from the OpenVSP Parasite Drag Tool, while induced drag, lift and lifting-surface moments are taken from the converged clean VSPAERO lifting model with the established mesh/wake uncertainty. No additional H2/H3 clean-body tuning is authorised; remaining validation effort transfers to 05_Stability.

## 7.3 Handoff to 05_Stability

Freeze the G1.6 files as diagnostic evidence; do not use the analysis-only FS 11.500 m TailRootFairing as production geometry.

Use the converged thin lifting configuration for the initial stability derivative campaign.

Prioritise β-sweep derivatives (CYβ, Clβ, Cnβ), followed by elevator, aileron and rudder control derivatives.

Use the clean-polar Cmα / CLα relationship for the lifting-system neutral-point estimate, then apply a separate analytical fuselage correction rather than importing the rejected thick-body H2 result.

Retain PDT as the parasite-drag source and avoid using mixed-model VSPAERO CDo/CDtot as complete-aircraft drag.

# 8. Analysis-file register

| Group | Files |
| --- | --- |
| G1.4 scripts | Tyche_R2T1_G1_4_Analysis.vspscript |
| G1.5 scripts | Tyche_R2T1_G1_5_Analysis.vspscript |
| G1.6 scripts | Tyche_R2T1_G1_6_Analysis.vspscript; Tyche_R2T1_G1_6_Analysis_H2.vspscript |
| G1.4 result CSVs | TRIAL.csv; H1_MODE2_MW_FUSE_FAIRING_A4.csv; H2_Mode_1.csv |
| G1.5 result CSVs | H2_Mode_1(1).csv; HT_TEST.csv; HT_Test_2.csv; H2_Test_2.csv; H2_NO_VT_TEST.csv |
| G1.6 result CSVs | HT_TEST_1.csv; HT_TEST_2(1).csv; HT_TEST_3.csv; HT_TEST_4.csv; H2_NO_VT_TEST_1.csv; H2_TEST_1.csv |
| Clean-aero production evidence | CA_G0_Clean_Lifting.csv; 01_G1_Strut_Sensitivity.csv; CA_G3_Repeatability_A4.csv |

END OF 04_Clean_Aero G1.4–G1.6 ANALYSIS LOG