TYCHE R2-T1
SM6.3B Munk-Multhopp Fuselage Stability Correction

Station-by-station analytical correction to the validated thin lifting-system neutral point

| Headline quantity | Result |
| --- | --- |
| SM6.3B disposition | PASS as an analytical calculation / FAIL against current aft-CG 10% MAC gate |
| Fuselage Cₘα correction | +0.3403 rad⁻¹ (+0.005939 deg⁻¹) |
| Corrected Cₘα | -0.8332 rad⁻¹ |
| Fuselage-corrected neutral point | FS 4.0289 m = 52.04% MAC |
| Static margin at FS 3.600 | 22.80% MAC |
| Static margin at FS 3.900 | 6.85% MAC — FAIL vs 10% floor |
| Aft-CG station for exactly 10% MAC | FS 3.8408 m = 42.04% MAC |

Engineering conclusion. The Munk-Multhopp fuselage correction moves the stick-fixed neutral point forward by 0.1139 m (6.06% MAC), from FS 4.1429 m to FS 4.0289 m. The current design aft-CG boundary at FS 3.900 m consequently retains only 6.85% MAC static margin and does not meet the present 10% MAC conceptual gate. This conclusion is robust to the 11–20-region discretization sensitivity performed below.

# 1. Scope and controlled analysis basis

SM6.3B applies an analytical Munk-Multhopp fuselage pitching-moment-slope correction to the VSPAERO thin lifting-system result. It does not reintroduce the rejected mixed thin/thick VSPAERO body solution. The output is therefore a conceptual analytical correction, not CFD, wind-tunnel, or flight-test validation.

| Controlled quantity | Value | Source |
| --- | --- | --- |
| Wing reference area, S | 30.000 m² | G5 / SM6.1 |
| Reference chord, c̄ | 1.881 m | G5 / SM6.1 |
| Wing span, b | 16.400 m | G1.3 |
| Aspect ratio, AR | 8.96533 | Derived |
| Moment reference, Xref | FS 3.750 m | SM6.1 |
| MAC leading edge | FS 3.050 m | G1.3 |
| Wing root LE / TE | FS 2.930079 / 5.290425 m | G1.3 |
| HT aerodynamic centre | FS 9.420166 m | G1.3 |
| TE-to-HT AC distance, lₕ | 4.129741 m | Derived |
| MainWing-only C_Lα,w | 4.88866 rad⁻¹ | SM6.3A |
| Lifting-system C_Lα | 5.61880 rad⁻¹ | SM6.1 |
| Lifting-system Cₘα | -1.17352 rad⁻¹ | SM6.1 |
| Validated lifting-system NP | FS 4.14286 m | SM6.1 / SM6.2 |

# 2. Analytical method

Gudmundsson identifies Munk-Multhopp as a conceptual-design method for estimating the destabilising pitching-moment-slope contribution of fuselages and nacelles. For the angle-of-attack-dependent term, the discrete form uses the local fuselage width, local upwash/downwash gradient and segment length. The wing-covered fuselage interval is assigned zero contribution in this implementation.

Cₘα,FUS = [1 / (36.5 S c̄)] Σ [ w_f,i² (∂β/∂α)_i Δx_i ]    (SM6.3B-1)

Equation (SM6.3B-1) returns Cₘα,FUS per degree because the factor 36.5 is the degree-form constant in the cited formulation. Conversion to radian⁻¹ is made by multiplying by 180/π.

(dε/dα)_w = 2 C_Lα,w / (π AR)    (SM6.3B-2)

(dε/dα)_w = 2(4.88866)/[π(8.96533)] = 0.34714    (SM6.3B-3)

For forward fuselage segments, x is the distance from the wing root leading edge to the segment centre. Upwash Curve 1 is used for x/c̄ ≥ 1 and Curve 2 for x/c̄ < 1, following the method’s application map.

(∂β/∂α)₁ = 1.4758 − 0.40485(x/c̄) + 0.09107(x/c̄)²    (SM6.3B-4)

(∂β/∂α)₂ = 1.74611(x/c̄)⁻⁰·⁴²⁵⁴    (SM6.3B-5)

For aft-body segments, x_i is measured aft from the wing root trailing edge and lₕ is the distance from that trailing edge to the horizontal-tail aerodynamic centre.

(∂β/∂α)_i = (x_i/lₕ)[1 − dε/dα]    (SM6.3B-6)

The neutral-point correction is applied additively to the VSPAERO lifting-system pitching-moment slope. No separate fuselage lift-curve-slope correction is introduced in this conceptual method.

x_NP,corr = x_ref − c̄ [ (Cₘα,LS + Cₘα,FUS) / C_Lα,LS ]    (SM6.3B-7)

# 3. Frozen G1.3 fuselage geometry

| G1.3 station | FS x (m) | Width (m) | Height (m) |
| --- | --- | --- | --- |
| Nose point | 0.0000 | 0.000 | 0.000 |
| Nose | 0.5375 | 0.800 | 0.900 |
| Cabin front | 2.3650 | 1.550 | 1.700 |
| Cabin rear | 5.3750 | 1.550 | 1.700 |
| Upsweep | 7.0950 | 1.250 | 1.340 |
| Tail boom | 9.1375 | 0.780 | 0.800 |
| Tail point | 10.7500 | 0.000 | 0.000 |

The station set is the frozen 10.750 m, seven-section rounded-rectangle G1.3 fuselage. The root-wing covered interval is FS 2.930079–5.290425 m. Widths between G1.3 stations are linearly interpolated for the numerical integration; the segment average width is the mean of the endpoint widths.

# 4. Station-by-station Munk-Multhopp integration

The baseline discretisation uses 14 regions: five forward-body segments, one wing-covered interval, and eight aft-body segments. Segment boundaries are aligned with the frozen G1.3 body stations wherever practical and subdivided only where needed to remain within the recommended 10–20-segment conceptual range.

| ID | Region | x₀ | x₁ | Δx | w_f | x/c or x/lₕ | ∂β/∂α | w_f²(∂β/∂α)Δx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F1 | Forward | 0.0000 | 0.5375 | 0.5375 | 0.4000 | 1.4148 | 1.0853 | 0.09334 |
| F2 | Forward | 0.5375 | 1.1467 | 0.6092 | 0.9250 | 1.1100 | 1.1386 | 0.59347 |
| F3 | Forward | 1.1467 | 1.7558 | 0.6092 | 1.1750 | 0.7862 | 1.9343 | 1.62677 |
| F4 | Forward | 1.7558 | 2.3650 | 0.6092 | 1.4250 | 0.4623 | 2.4244 | 2.99890 |
| F5 | Forward | 2.3650 | 2.9301 | 0.5651 | 1.5500 | 0.1502 | 3.9112 | 5.30982 |
| W | Wing-covered | 2.9301 | 5.2904 | 2.3603 | 1.5500 | — | 0.0000 | 0.00000 |
| A1 | Aft | 5.2904 | 5.3750 | 0.0846 | 1.5500 | 0.0102 | 0.0067 | 0.00136 |
| A2 | Aft | 5.3750 | 6.2350 | 0.8600 | 1.4750 | 0.1246 | 0.0813 | 0.15221 |
| A3 | Aft | 6.2350 | 7.0950 | 0.8600 | 1.3250 | 0.3328 | 0.2173 | 0.32809 |
| A4 | Aft | 7.0950 | 8.1163 | 1.0213 | 1.1325 | 0.5606 | 0.3660 | 0.47940 |
| A5 | Aft | 8.1163 | 9.1375 | 1.0212 | 0.8975 | 0.8079 | 0.5275 | 0.43389 |
| A6 | Aft | 9.1375 | 9.6750 | 0.5375 | 0.6500 | 0.9966 | 0.6507 | 0.14776 |
| A7 | Aft | 9.6750 | 10.2125 | 0.5375 | 0.3900 | 1.1268 | 0.7356 | 0.06014 |
| A8 | Aft | 10.2125 | 10.7500 | 0.5375 | 0.1300 | 1.2569 | 0.8206 | 0.00745 |

Integration totals: forward body = 10.622298 m³; aft body = 1.610302 m³; total = 12.232600 m³. The forward body supplies 86.8% of the total destabilising integral and the aft body 13.2%.

# 5. Fuselage pitching-moment-slope result

Cₘα,FUS = 12.232600/[36.5(30.000)(1.881)] = +0.005939 deg⁻¹    (SM6.3B-8)

Cₘα,FUS = +0.34028 rad⁻¹    (SM6.3B-9)

The correction is positive and therefore destabilising. It is about 29.0% of the magnitude of the original lifting-system Cₘα. It also exceeds the previously established +0.1636 rad⁻¹ destabilising budget for retaining 10% MAC at FS 3.900 by a factor of 2.08.

# 6. Corrected neutral point and static-margin gate

Cₘα,corr = -1.17352 + 0.34028 = -0.83324 rad⁻¹    (SM6.3B-10)

x_NP,corr = 3.750 − 1.881[-0.83324/5.61880] = 4.02894 m    (SM6.3B-11)

h_N,corr = (4.02894 − 3.050)/1.881 = 0.52044 = 52.04% MAC    (SM6.3B-12)

| Item | FS x (m) | % MAC | Assessment |
| --- | --- | --- | --- |
| Lifting-system NP (before body correction) | 4.14286 | 58.10% | Reference |
| Fuselage-corrected NP | 4.02894 | 52.04% | Accepted SM6.3B estimate |
| Forward design CG | 3.600 | 29.24% | SM = 22.80% MAC |
| Aft design CG | 3.900 | 45.19% | SM = 6.85% MAC — FAIL |
| NP required for 10% at aft CG | 4.0881 | 55.19% | Required |
| Aft CG allowed for 10% with corrected NP | 3.8408 | 42.04% | Provisional aft limit |

10% MAC gate decision: FAIL. At FS 3.900 m, the fuselage-corrected static margin is 6.85% MAC, 3.15 percentage points below the 10% target. Equivalently, the corrected neutral point is 0.0592 m forward of the FS 4.0881 m neutral-point location required to retain 10% MAC at the current aft-CG boundary. Holding the corrected neutral point fixed would require the aft-CG design boundary to move forward to about FS 3.8408 m (42.04% MAC).

# 7. Discretisation sensitivity and robustness

| Mesh | Total regions | Forward | Aft | Cₘα,FUS (rad⁻¹) | x_NP (m) | SM @ FS3.90 |
| --- | --- | --- | --- | --- | --- | --- |
| Coarse | 11 | 4 | 6 | 0.33104 | 4.03204 | 7.02% |
| Baseline | 14 | 5 | 8 | 0.34028 | 4.02894 | 6.85% |
| Fine | 20 | 8 | 11 | 0.35815 | 4.02296 | 6.54% |

All three integrations remain inside the method’s 10–20-region conceptual range. The resulting aft-CG static margin spans only about 6.54–7.02% MAC and therefore remains below 10% for every tested discretisation. The pass/fail conclusion is not a segmentation artefact.

# 8. SM6.3B disposition and handoff

| Gate / quantity | Result | Disposition |
| --- | --- | --- |
| Frozen 10.75 m G1.3 fuselage stations used | Yes | PASS |
| SM6.3A wing-only C_Lα,w used | 4.88866 rad⁻¹ | PASS |
| Wing downwash scale | 0.34714 | PASS |
| 14-region station-based integration | Completed | PASS |
| Cₘα,FUS sign | Positive / destabilising | Physically expected |
| Cₘα,FUS | +0.34028 rad⁻¹ | ACCEPT analytical estimate |
| Corrected x_NP | FS 4.02894 m / 52.04% MAC | ACCEPT fuselage-corrected |
| Forward-CG static margin | 22.80% MAC | PASS |
| Aft-CG static margin | 6.85% MAC | FAIL vs 10% floor |
| Discretisation sensitivity | 6.54–7.02% MAC aft SM | FAIL robust |
| Overall SM6.3B | Calculation complete | CLOSED / FAIL current aft-CG gate |

Closure statement. SM6.3B is CLOSED as a completed conceptual analytical correction. Using the frozen 10.75 m G1.3 fuselage and the measured MainWing-only C_Lα,w = 4.88866 rad⁻¹, the station-based Munk-Multhopp integration gives Cₘα,FUS = +0.3403 rad⁻¹. Applied to the validated lifting-system slopes, this moves the stick-fixed neutral point from FS 4.1429 m (58.10% MAC) to FS 4.0289 m (52.04% MAC). Static margin becomes 22.80% MAC at FS 3.600 and only 6.85% MAC at FS 3.900. Therefore the current FS 3.900 aft-CG boundary fails the 10% MAC conceptual stability gate. The next step is SM6.4: compare this corrected stability boundary with the actual R2-T1 mass/loading CG cloud and either prove that operational loading remains forward of approximately FS 3.841 m or reopen the aft-CG/tail-sizing design gate.

Scope boundary. The numerical correction above is for the fuselage only. Nacelle and powered-propeller normal-force contributions are not included in SM6.3B. Consequently, the quoted neutral point is explicitly the fuselage-corrected lifting-system neutral point, not a final certification-level complete-aircraft neutral point. These omitted effects should remain traceable in the final stability uncertainty register.

# References and evidence

[1] S. Gudmundsson, General Aviation Aircraft Design: Applied Methods and Procedures, 1st ed., Butterworth-Heinemann/Elsevier, 2014, Chapter 11 and Appendix C1.6.3, “Contribution of Fuselage and Nacelles.”

[2] Tyche R2-T1 G1.3 analysis-ready geometry script, Tyche_R2T1_G1_3_Analysis.vspscript, frozen 10.750 m seven-station fuselage and 5.90 m c/4-to-c/4 tail-arm definition.

[3] G5_CGSM_ALPHA_M2_P2(1).csv, SM6.1 five-point local static-stability sweep: C_Lα = 5.61880 rad⁻¹, Cₘα = −1.17352 rad⁻¹, x_NP,LS = 4.14286 m.

[4] G5_CGSM_XREF390_CHECK.csv, SM6.2 moment-reference invariance audit.

[5] G5_SM63_WING_ONLY_ALPHA_M2_P2.csv, SM6.3A MainWing-only support run: C_Lα,w = 4.88866 rad⁻¹.

[6] Tyche R2-T1 04_Clean_Aero G1.4–G1.6 analysis closure: production stability work retains the converged thin lifting-system model and applies a separate analytical fuselage correction rather than importing the rejected mixed thin/thick-body solution.