# TYCHE R2-T1

SM6.3B Verification and Integrated SM6.4 Mass-Properties / CG-Envelope Closure

Tyche Design Team

21 August 2026

Analysis release: SM6.3B + SM6.4A-C
Configuration: Frozen 10.750 m G1.3 fuselage / G5 lifting-system stability model
Purpose: Final integrated longitudinal static-stability and loading-envelope record before 07_VMC_Control_Closure

| Headline quantity | Integrated result |
| --- | --- |
| SM6.3B equation and central arithmetic | PASS - independently reproduced |
| Fuselage correction, Cmα,FUS | +0.34028rad−1 |
| Fuselage-corrected neutral point | FS 4.02894 m = 52.04% MAC |
| U20 screening neutral point / 10% aft limit | FS 4.02588 m / FS 3.83778 m |
| Geometry-based OEW CG | FS 3.42993 m = 20.20% MAC |
| Nominal loading-cloud extent | FS 3.34618-3.70312 m |
| Most aft nominal U20 static margin | 17.16% MAC - PASS |
| Independent-uniform 95% aft screen | FS 3.76080 m; 14.09% MAC - PASS |
| All-at-bounds deterministic aft screen | FS 3.91324 m; 5.99% MAC - HOLD / FAIL release |
| Previous FS 3.600 forward boundary | Not contained; forward-control / gear gate reopened |
| Overall 06_CG_Static_Margin | Conditional analysis PASS; operational release HOLD |
| Handoff | Proceed to 07_VMC_Control_Closure with the controlled cases in Section 11 |

Engineering conclusion. The verified correction invalidates FS 3.900 m, but the geometry-based nominal cloud ends at FS 3.70312 m with 17.16% MAC static margin on U20; a tail resize is not indicated. Operational release remains on HOLD because several arms are not CAD centroids and the all-at-bounds screen fails. Valid light/front-row states reach FS 3.34618 m and about 26.7% nose load, making forward controllability and gear reaction the next longitudinal gate.

# 1. Verification scope and controlled evidence

This record combines the verified SM6.3B fuselage correction with a completed conceptual SM6.4 mass-arm and loading-cloud calculation. The SM6.3B audit is retained so that the loading result remains connected to the exact neutral-point basis. The new SM6.4 work uses the 30-line operating-empty-mass statement in the design report, the frozen G1.3 stations, the cabin and propulsion geometry recorded in the report and analysis chat, and explicit engineering arm bounds. No mass is deleted and no credit is counted twice.

| Audit item | Finding | Disposition |
| --- | --- | --- |
| Governing Munk-Multhopp strip integral | Matches Gudmundsson Appendix C1 discrete form | PASS |
| Degree/radian conversion | 36.5 equation returns per degree; 180/π applied | PASS |
| Frozen fuselage interpolation | Piecewise-linear width; endpoint-average strip width | PASS |
| 14-region station arithmetic | Independently reproduced | PASS |
| Alternative discretisations | U11, U14 and U20 boundaries explicitly defined | PASS |
| OEW mass total | 30-line ledger sums to 1,327.0 kg | PASS |
| OEW arm model | Geometry-based and source-graded; not measured CAD centroids | CONCEPTUAL |
| Loading cloud | All passenger subsets and endpoint cargo/fuel states enumerated | PASS |
| Complete-aircraft aerodynamic model | Nacelle, powered-propeller and body-lift corrections remain open | HOLD |

## 1.1 Controlled aerodynamic quantities

| Quantity | Value | Evidence |
| --- | --- | --- |
| Wing reference area, S | 30.000 m² | G5 / SM6.1 |
| Mean aerodynamic chord, c | 1.881 m | G5 / SM6.1 |
| Wing span, b | 16.400 m | G1.3 |
| Aspect ratio, AR | 8.96533 | Derived |
| Moment reference, xref | FS 3.750 m | SM6.1 |
| MAC leading edge, xLE,MAC | FS 3.050 m | G1.3 |
| Wing root LE / TE | FS 2.930079 / 5.290425 m | G1.3 |
| Horizontal-tail aerodynamic centre | FS 9.420166 m | G1.3 |
| MainWing-only CLα,w | 4.88866 rad⁻¹ | SM6.3A |
| Lifting-system CLα | 5.61880 rad⁻¹ | SM6.1 |
| Lifting-system Cmα | -1.17352 rad⁻¹ | SM6.1 |
| Lifting-system neutral point | FS 4.14286 m | SM6.1 / SM6.2 |

# 2. Verified Munk-Multhopp methodology

Gudmundsson's Appendix C1 implements the Munk-Multhopp fuselage or nacelle correction by dividing the body into forward, wing-covered and aft regions. The angle-of-attack-dependent pitching-moment slope is computed from local body width, local flow-angle gradient and segment length. With all dimensions in metres, the dimensional ratio is unchanged.

Equation (SM6.3B-1):

Cmα,FUS,deg=136.5Sciwf,i2∂β∂αiΔxi

Equation (SM6.3B-2):

Cmα,FUS,rad=180πCmα,FUS,deg

The 36.5 form returns slope per degree; the conversion in Equation (SM6.3B-2) is therefore required.

## 2.1 Wing downwash input

Equations (SM6.3B-3) and (SM6.3B-4) are:

AR=b2S=16.4230.0=8.96533

dϵdα=2CLα,wπAR=24.88866π8.96533=0.347139

This is the conceptual elliptical-wing approximation used only in the aft-region relation; it is not applied uniformly to the forward body.

## 2.2 Forward and aft flow-angle gradients

For forward-body stations, x is the positive distance from the wing root leading edge to the strip midpoint. Curve 1 applies for x/c≥1 and Curve 2 for x/c<1.

∂β∂α1=1.4758−0.40485xc+0.09107xc2(SM6.3B-5)

∂β∂α2=1.74611xc−0.4254(SM6.3B-6)

For aft-body stations, xi is the distance aft of the wing root trailing edge and lh is the root-TE to horizontal-tail aerodynamic-centre distance.

∂β∂αi=xilh1−dϵdα(SM6.3B-7)

The root wing-covered interval FS 2.930079-5.290425 m is retained as a zero-contribution row, consistent with the source implementation map. The Curve 2 singularity as x/c→0 is integrable but creates mesh-placement sensitivity; Section 6 audits it explicitly.

## 2.3 Neutral point and static margin

Cmα,corr=Cmα,LS+Cmα,FUS(SM6.3B-8)

xNP,corr=xref−cCmα,corrCLα,LS(SM6.3B-9)

SMxCG=xNP,corr−xCGc(SM6.3B-10)

No separate fuselage lift-curve-slope term is added in this conceptual implementation. The result is therefore a fuselage-moment-corrected lifting-system neutral point, not a complete-aircraft CFD-equivalent neutral point.

# 3. Frozen 10.750 m G1.3 fuselage geometry

| G1.3 station | FS x (m) | Width (m) | Height (m) |
| --- | --- | --- | --- |
| Nose point | 0.0000 | 0.000 | 0.000 |
| Nose | 0.5375 | 0.800 | 0.900 |
| Cabin front | 2.3650 | 1.550 | 1.700 |
| Cabin rear | 5.3750 | 1.550 | 1.700 |
| Upsweep | 7.0950 | 1.250 | 1.340 |
| Tail boom | 9.1375 | 0.780 | 0.800 |
| Tail point | 10.7500 | 0.000 | 0.000 |

Widths are linearly interpolated between the frozen stations. Each Munk-Multhopp strip uses the mean of its endpoint widths. Height is retained for geometry traceability and for the fuselage structural-centroid proxy in Section 7, but it does not enter the width-based pitching-moment equation.

# 4. Independent reproduction of the 14-region integration

The station-aligned baseline uses five forward segments, one zero wing-covered interval and eight aft segments. All tabulated values were recomputed from the controlled inputs.

## 4.1 Forward and wing-covered regions

| ID | x0-x1 (m) | Δx | wf | Ratio | ∂β/∂α | Term (m³) |
| --- | --- | --- | --- | --- | --- | --- |
| F1 | 0.0000-0.5375 | 0.5375 | 0.4000 | 1.4148 | 1.0853 | 0.09334 |
| F2 | 0.5375-1.1467 | 0.6092 | 0.9250 | 1.1100 | 1.1386 | 0.59351 |
| F3 | 1.1467-1.7558 | 0.6091 | 1.1750 | 0.7862 | 1.9343 | 1.62659 |
| F4 | 1.7558-2.3650 | 0.6092 | 1.4250 | 0.4623 | 2.4243 | 2.99902 |
| F5 | 2.3650-2.9301 | 0.5651 | 1.5500 | 0.1502 | 3.9112 | 5.30982 |
| W | 2.9301-5.2904 | 2.3603 | 1.5500 | - | 0.0000 | 0.00000 |

The forward-body integral is 10.622273 m³. The wing-covered interval contributes zero by definition in the selected implementation.

## 4.2 Aft-body regions

| ID | x0-x1 (m) | Δx | wf | Ratio | ∂β/∂α | Term (m³) |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | 5.2904-5.3750 | 0.0846 | 1.5500 | 0.0102 | 0.0067 | 0.00136 |
| A2 | 5.3750-6.2350 | 0.8600 | 1.4750 | 0.1246 | 0.0813 | 0.15221 |
| A3 | 6.2350-7.0950 | 0.8600 | 1.3250 | 0.3328 | 0.2173 | 0.32809 |
| A4 | 7.0950-8.1163 | 1.0213 | 1.1325 | 0.5606 | 0.3660 | 0.47942 |
| A5 | 8.1163-9.1375 | 1.0212 | 0.8975 | 0.8079 | 0.5275 | 0.43387 |
| A6 | 9.1375-9.6750 | 0.5375 | 0.6500 | 0.9966 | 0.6507 | 0.14776 |
| A7 | 9.6750-10.2125 | 0.5375 | 0.3900 | 1.1268 | 0.7356 | 0.06014 |
| A8 | 10.2125-10.7500 | 0.5375 | 0.1300 | 1.2569 | 0.8206 | 0.00745 |

The aft-body integral is 1.610303 m³ and the total is 12.232575 m³. The forward body supplies 86.8% of the positive destabilising integral.

# 5. Verified central SM6.3B result

Cmα,FUS=12.23257536.530.0001.881=+0.005939deg−1(SM6.3B-11)

Cmα,FUS=+0.34028rad−1(SM6.3B-12)

Cmα,corr=−1.17352+0.34028=−0.83324rad−1(SM6.3B-13)

xNP,corr=3.750−1.881−0.833245.61880=4.02894m(SM6.3B-14)

| Item | FS x (m) | % MAC | Assessment |
| --- | --- | --- | --- |
| Lifting-system neutral point | 4.14286 | 58.10% | Reference-invariant thin model |
| Fuselage-corrected neutral point | 4.02894 | 52.04% | Central conceptual estimate |
| Former forward design CG | 3.60000 | 29.24% | SM = 22.80% |
| Former aft design CG | 3.90000 | 45.19% | SM = 6.86% - FAIL |
| Neutral point required for 10% at FS 3.900 | 4.08810 | 55.19% | Required |
| Central aft CG for exactly 10% | 3.84084 | 42.04% | Central limit |

The former FS 3.900 m boundary is 0.0592 m too far aft and misses the 10%-MAC floor by 3.14 percentage points. It is not used as an acceptable boundary in the loading assessment.

# 6. Discretisation and aerodynamic model-form audit

The original station-aligned solution is S14. Reproducible uniform alternatives are U11 = four forward + one wing-covered + six aft regions, U14 = five + one + eight, and U20 = eight + one + eleven.

| Case | Definition | Regions | Cmα,FUS | xNP (m) | SM at 3.900 | 10% aft limit (m) |
| --- | --- | --- | --- | --- | --- | --- |
| U11 | Uniform 4 F + 1 W + 6 A | 11 | 0.32977 | 4.03246 | 7.04% | 3.84436 |
| U14 | Uniform 5 F + 1 W + 8 A | 14 | 0.33711 | 4.03000 | 6.91% | 3.84190 |
| S14 | Station-aligned 5 F + 1 W + 8 A | 14 | 0.34028 | 4.02894 | 6.86% | 3.84084 |
| U20 | Uniform 8 F + 1 W + 11 A | 20 | 0.34943 | 4.02588 | 6.69% | 3.83778 |
| Fine quadrature | Continuous-width midpoint warning | - | 0.38902 | 4.01263 | 5.99% | 3.82453 |

Every defined 10-to-20-region mesh fails FS 3.900. U20 is retained as the conservative reproducible screen. The fine quadrature resolves more of the integrable near-leading-edge singularity and is retained as a numerical warning, not substituted for the source strip method.

## 6.1 Model-form boundary

- Nacelle Munk-Multhopp terms are not included because complete station shapes and a confirmed datum were not in the controlled input set.

- Powered-propeller normal-force derivatives are not included.

- A separate fuselage lift-curve-slope contribution is not included; the denominator remains the lifting-system CLα.

- Classical high-wing model-form uncertainty remains open; numerical reproduction is not aerodynamic validation.

The selected central value is therefore a conceptual fuselage-moment correction. The U20 result is a conservative numerical screen within that same model, not a certification-level complete-aircraft neutral point.

# 7. SM6.4A - component mass-arm model

The design report supplies the 30-line mass statement but not longitudinal centroids. SM6.4A therefore constructs a first-order mass-properties ledger from the frozen geometry. It is suitable for conceptual design decisions because every assumption is explicit and bounded; it is not a substitute for CAD mass-property export or weighed-aircraft data.

## 7.1 Governing mass-property equations

M=imi,Mx=imixi,xCG=MxM(SM6.4A-1)

hCG=xCG−xLE,MACc(SM6.4A-2)

Growth allowance and integrated credits are accounting adjustments rather than physical point masses. Both are placed at the physical-subtotal CG so that neither creates a fictitious moment shift.

## 7.2 Arm-construction rules and evidence grades

| Grade | Meaning | Use in this release |
| --- | --- | --- |
| A | Direct frozen station or chordwise geometry | Propeller, wing devices, empennage |
| B | Derived centroid within a controlled installation envelope | Engines, fuselage, gear, ESS, cabin |
| C | Engineering allocation for a distributed or incompletely packaged system | Wiring, controls, avionics, equipment |
| N/A | Accounting row placed at physical-subtotal CG | Growth and integrated credits |

Key constructions are:

- Wing-box arm: xLE,MAC+0.40c=3.8024 m.

- Fuselage shell proxy: perimeter-weighted frozen-station centroid xshell=4.8468 m; the combined shell/carry-through row is assigned FS 4.65 m with a ±0.35 m bound.

- Landing-gear group: a transparent 25.0 kg nose / 87.2 kg main-and-brake allocation gives 250.75+87.24.29/112.2=3.5003 m.

- Three two-abreast row centres: FS 2.70, 3.65 and 4.60 m within the frozen FS 2.365-5.375 cabin.

- Wing fuel arm: FS 3.75 m, close to the wing structural and aerodynamic reference.

- Cargo restraint zone: FS 3.35-5.15 m; both endpoints are screened.

## 7.3 Controlled 30-line preliminary ledger

| No. | Mass item | Mass (kg) | FS arm (m) | Moment (kg·m) | Arm bound (m) | Grade |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Combustion engines, installed | 256.0 | 2.100 | 537.6 | ±0.20 | B |
| 2 | CF/PAEK wing torsion box | 190.1 | 3.802 | 722.8 | ±0.15 | A |
| 3 | Fuselage and wing carry-through | 132.1 | 4.650 | 614.3 | ±0.35 | B |
| 4 | Landing gear and brakes | 112.2 | 3.500 | 392.7 | ±0.20 | B |
| 5 | Electric machines/inverters/cooling | 72.0 | 2.250 | 162.0 | ±0.25 | B |
| 6 | ESS installed | 71.1 | 3.700 | 263.1 | ±0.15 | B |
| 7 | Double-slotted Fowler system | 61.9 | 4.550 | 281.6 | ±0.15 | A |
| 8 | Propellers and governors | 58.7 | 1.350 | 79.2 | ±0.10 | A |
| 9 | Design growth allowance | 52.0 | 3.430 | 178.4 | ±0.00 | N/A |
| 10 | Cabin furnishings and seats | 46.0 | 3.650 | 167.9 | ±0.35 | B |
| 11 | Avionics | 42.0 | 2.250 | 94.5 | ±0.30 | C |
| 12 | Low-voltage electrical system | 34.0 | 3.300 | 112.2 | ±0.50 | C |
| 13 | Horizontal tail | 30.8 | 9.480 | 292.0 | ±0.15 | A |
| 14 | Primary flight controls | 24.0 | 5.100 | 122.4 | ±1.20 | C |
| 15 | Retractable slat system | 22.0 | 3.160 | 69.5 | ±0.15 | A |
| 16 | Fuel system | 17.3 | 3.750 | 64.9 | ±0.35 | B |
| 17 | Hybrid couplings | 16.0 | 2.050 | 32.8 | ±0.20 | B |
| 18 | Vertical tail | 15.8 | 9.550 | 150.9 | ±0.20 | A |
| 19 | Fire protection | 13.0 | 2.750 | 35.8 | ±0.60 | C |
| 20 | High-voltage distribution | 12.0 | 3.200 | 38.4 | ±0.75 | C |
| 21 | Cargo handling and restraints | 11.0 | 4.100 | 45.1 | ±0.50 | B |
| 22 | Operating fluids | 11.0 | 2.200 | 24.2 | ±0.35 | B |
| 23 | Thermal-management hardware | 10.0 | 2.400 | 24.0 | ±0.50 | C |
| 24 | Instruments and miscellaneous | 14.0 | 2.300 | 32.2 | ±0.40 | C |
| 25 | Marine survival equipment | 9.0 | 4.000 | 36.0 | ±0.60 | C |
| 26 | Engine controls | 8.0 | 2.800 | 22.4 | ±0.60 | C |
| 27 | Emergency equipment | 8.0 | 3.750 | 30.0 | ±0.65 | C |
| 28 | CFRP wing struts | 6.4 | 3.720 | 23.8 | ±0.30 | B |
| 29 | Ailerons | 1.6 | 4.550 | 7.3 | ±0.15 | A |
| 30 | Integrated structural/system credits | -31.0 | 3.430 | -106.3 | ±0.00 | N/A |
|  | Operating-empty total | 1,327.0 | 3.42993 | 4,551.5 |  |  |

Thus:

xCG,OEW=4551.52081327.0=3.42993m=20.20%MAC(SM6.4A-3)

This is 0.17007 m forward of the former FS 3.600 design boundary. It is a calculated conceptual centroid, not a measured result.

# 8. SM6.4B - loading-cloud construction and results

## 8.1 Loading definitions

| Variable block | Controlled values |
| --- | --- |
| Occupants | Pilot + five passenger stations; 86 kg each |
| Seat arms | Pilot/P1: 2.70 m; P2/P3: 3.65 m; P4/P5: 4.60 m |
| Cargo | 0-200 kg, screened at FS 3.35 and FS 5.15 |
| Fuel arm | FS 3.75 m |
| Fuel states | 109.8, 107.3, 106.1, 98.1, 23.2, 20.8, 18.8, 13.7, 5.1 and 0 kg |
| Mass gates | 2,000 kg MTOM; 1,900 kg hot-high planned landing mass |

For fixed load arm xj, the CG varies monotonically with the variable load mass:

∂xCG∂mj=xj−xCGM(SM6.4B-1)

Therefore cargo and fuel endpoint masses are sufficient to identify the extrema for each passenger subset. The enumeration covers every subset of the five passenger positions with the pilot aboard, both cargo-zone endpoints, zero and maximum allowable cargo, all controlled fuel states, and the 2,000 kg cap. This produces 1,280 flyable endpoint records. It deliberately permits the conservative combination of aft passengers and aft-end cargo even where final cabin-volume rules may make that combination incompatible; this prevents the conceptual screen from hiding an aft case behind an unverified packing assumption.

## 8.2 Critical loading cases

| Case | Occupants / cargo / fuel | Mass (kg) | FS CG (m) | % MAC | U20 SM | Nose load |
| --- | --- | --- | --- | --- | --- | --- |
| OEW | No payload / no dispatch fuel | 1,327.0 | 3.42993 | 20.20% | 31.68% | 24.30% |
| Most-forward endpoint | Pilot + P1; zero cargo/fuel | 1,499.0 | 3.34618 | 15.75% | 36.14% | 26.66% |
| Forward take-off | Pilot + P1; 109.8 kg fuel | 1,608.8 | 3.37374 | 17.21% | 34.67% | 25.88% |
| Delivered 505 km mission | Six occupants; 47.2 kg aft cargo; 109.8 kg fuel | 2,000.0 | 3.54488 | 26.31% | 25.57% | 21.05% |
| Most-aft MTOM | Pilot + P2-P5; 133.2 kg aft cargo; 109.8 kg fuel | 2,000.0 | 3.65023 | 31.91% | 19.97% | 18.07% |
| Most-aft 1,900 kg landing | Pilot + P2-P5; 122.2 kg aft cargo; 20.8 kg fuel | 1,900.0 | 3.63687 | 31.20% | 20.68% | 18.45% |
| Most-aft nominal cloud | Pilot + P4/P5; 200 kg aft cargo; 109.8 kg fuel | 1,894.8 | 3.70312 | 34.72% | 17.16% | 16.58% |
| Full RFP arithmetic | Six occupants; 200 kg aft cargo; 109.8 kg fuel | 2,152.8 | 3.65880 | 32.37% | 19.51% | 17.83% |

The 2,152.8 kg RFP row is retained only as a traceability calculation; it is not a flyable case and remains a mass-compliance failure. All flyable nominal aft points remain forward of FS 3.83778 m. The most-aft point has:

SMU20,aft=4.02588−3.703121.881=0.1716=17.16%MAC(SM6.4B-2)

Its distance from the exact U20 10% boundary is 0.13466 m, or 7.16% MAC. It is also 0.07823 m forward of the 13%-MAC buffered target FS 3.78135 m, retaining 4.16% MAC beyond that buffered target.

## 8.3 Nominal gate result

| Gate | Result | Disposition |
| --- | --- | --- |
| Every flyable state ≤ 2,000 kg | Enforced during enumeration | PASS |
| Nominal aft containment | xCG,max=3.70312<3.83778 m | PASS |
| U20 static margin at nominal aft case | 17.16% MAC | PASS |
| 3%-MAC analysis buffer beyond 10% | 4.16% MAC retained | PASS |
| Former FS 3.900 aft boundary | Not used | CLOSED / REJECTED |
| Need for immediate tail resize | Not indicated by nominal cloud | NO |

# 9. SM6.4C - uncertainty and robust-release audit

The arm bounds in Section 7 are engineering half-ranges. Two complementary screens are reported. The first places every item simultaneously at its adverse bound. This is deterministic but extremely conservative because it assumes all unrelated installation errors align in the same direction. The second treats the half-ranges as independent uniform variables solely for a conceptual probabilistic sensitivity; it is not a substitute for a validated statistical tolerance model.

For the all-at-bounds screen:

ΔMbox=imiui,ΔxCG,box=ΔMboxM(SM6.4C-1)

For the independent-uniform sensitivity:

σM=imiui23,Δx95=1.96σMM(SM6.4C-2)

| Uncertainty screen | Aft CG (m) | U20 static margin | 10% gate | 13% buffered target |
| --- | --- | --- | --- | --- |
| Nominal | 3.70312 | 17.16% | PASS | PASS |
| Independent-uniform 95% | 3.76080 | 14.09% | PASS | PASS by 0.02055 m |
| Simultaneous all-at-bounds | 3.91324 | 5.99% | FAIL | FAIL |

The deterministic bound uses a total adverse moment allowance of 398.135 kg·m for the critical aft case. The independent-uniform 95% screen uses Δx95=0.05768 m. The nominal and 95% conceptual screens show ample reason to retain the current tail for the next control analysis, but the all-at-bounds failure prevents calling the operational CG envelope fully released. CAD centroids and installation drawings should replace Grade C arms first because distributed flight controls, electrical systems, fire protection and equipment dominate the uncertainty ledger.

# 10. Forward-CG, trim and landing-gear consequence

The former FS 3.600 lower boundary is not a minimum static-stability requirement. Moving forward increases static margin, but it increases elevator/tail load and nose-gear reaction. The geometry-based cloud therefore changes the forward closure question from “is the aircraft stable?” to “can it rotate, trim, flare and go around, and are the ground reactions acceptable?”

The static nose-load fraction for a tricycle gear is:

WNW=xMG−xCGxMG−xNG=4.29−xCG4.29−0.75(SM6.4C-3)

| Forward case | Mass (kg) | FS CG (m) | U20 SM | Static nose load | Disposition |
| --- | --- | --- | --- | --- | --- |
| OEW | 1,327.0 | 3.42993 | 31.68% | 24.30% | Ground handling only; crew absent |
| Pilot + P1, dispatch fuel | 1,608.8 | 3.37374 | 34.67% | 25.88% | Take-off rotation / gear HOLD |
| Pilot + P1, landing fuel | 1,519.8 | 3.35170 | 35.84% | 26.51% | Flare / go-around / gear HOLD |
| Pilot + P1, zero fuel | 1,499.0 | 3.34618 | 36.14% | 26.66% | Ground loading endpoint |

The design report's previous FS 3.60-3.90 analysis corresponded to about 19.5%-11.0% nose load. The revised preliminary ledger therefore produces valid front-loaded states above that former nose-load band. These points are not rejected merely because they lie forward of FS 3.600; instead, FS 3.34618 becomes the conceptual forward candidate and is carried into a dedicated elevator/rotation/gear release. Operational loading restrictions may be used only if the full required mission and safe loading/unloading sequence remain achievable.

# 11. Final dispositions and handoff to 07_VMC_Control_Closure

| Item | Disposition | Controlled statement |
| --- | --- | --- |
| SM6.3B source and arithmetic | PASS / CLOSED | Cmα,FUS=+0.34028 rad⁻¹; central NP FS 4.02894 m |
| Former FS 3.900 aft gate | FAIL / CLOSED | 6.86% MAC; no longer accepted |
| SM6.4A mass-arm arithmetic | PASS - conceptual | 1,327.0 kg, 4,551.5 kg·m, OEW CG FS 3.42993 m |
| SM6.4B nominal loading cloud | PASS | 1,280 endpoint records; aft limit FS 3.70312 m |
| Nominal aft static margin | PASS | 17.16% MAC on U20 screen |
| Independent-uniform 95% screen | PASS - sensitivity only | FS 3.76080 m; 14.09% MAC |
| All-at-bounds arm screen | FAIL / HOLD | FS 3.91324 m; measured/CAD centroids required |
| Forward loading containment | HOLD | Valid cases reach FS 3.34618 m; controls and gear must release |
| Tail resize decision | DEFER / NOT INDICATED | Nominal aft cloud retains 7.16% MAC beyond the 10% gate |
| Overall 06_CG_Static_Margin | CONDITIONAL ANALYSIS PASS / RELEASE HOLD | Adequate basis to proceed; not an approved operational envelope |

## 11.1 Controlled VMC input cases

07_VMC_Control_Closure should no longer use FS 3.900 as an assumed real loading point. It should screen the following states and retain the U20 limit only as an uncertainty/sensitivity bound.

| Case ID | Mass (kg) | FS CG (m) | Configuration meaning | Use in VMC work |
| --- | --- | --- | --- | --- |
| VMC-A | 1,894.8 | 3.70312 | Most-aft nominal conservative departure screen; dispatch fuel | Primary actual-loading case |
| VMC-B | 2,000.0 | 3.65023 | Most-aft MTOM case; five occupants + 133.2 kg aft cargo | MTOM comparison |
| VMC-C | 1,900.0 | 3.63687 | Most-aft planned landing-mass case | Approach/go-around comparison |
| VMC-U | Variable | 3.83778 | U20 exact 10% boundary; not an actual nominal loading point | Conservative CG sensitivity only |

The VMC analysis may therefore proceed without an immediate longitudinal geometry change. It must still solve simultaneous yaw, roll and sideslip equilibrium with finite rudder/aileron limits and left/right critical-engine cases. In parallel, the later forward-control closure must use FS 3.37374 m at 1,608.8 kg for take-off rotation and FS 3.35170 m at 1,519.8 kg for landing/flare/go-around screening. The 30-line arm ledger must be updated when CAD centroids become available; any updated point exceeding FS 3.83778 m reopens the tail/layout gate automatically.

Final closure statement. SM6.3B is closed as a verified conceptual fuselage correction. SM6.4 now contains a reproducible geometry-based component ledger and complete endpoint loading cloud. The nominal aft-CG design question passes and does not justify resizing the horizontal tail before VMC analysis. The operational release remains on HOLD because the hard arm-bound screen fails and the forward loading edge lies outside the former design range. The controlled next action is 07_VMC_Control_Closure, using VMC-A through VMC-U above, followed by forward-CG elevator/rotation/flare and landing-gear closure.

# References and controlled evidence

[1] S. Gudmundsson, General Aviation Aircraft Design: Applied Methods and Procedures, 1st ed., Butterworth-Heinemann/Elsevier, 2014, Sections 6.6.5, 6.6.11-6.6.12 and Chapter 11 / Appendix C1.

[2] H. Multhopp, Aerodynamics of the Fuselage, NACA Technical Memorandum 1036, 1942.

[3] A. ul Haque, W. Asrar and E. Sulaeman, “A Revisit to Munk-Multhopp's Method for Estimating the Pitching Moment of a Fuselage Immersed in a High Wing Flow,” AIAC-2017-066, 2017.

[4] Royal Aeronautical Society, International Light Aircraft Design Competition Rules 2025/26, stability-and-control and Final Design Report mass-breakdown/validation requirements.

[5] Tyche R2-T1 Conceptual Design Report, controlled 30-line Table 6.1 mass statement, Table 5.3 fuel statement, cabin arrangement, landing-gear stations and configuration definition.

[6] Tyche R2-T1 G1.3 analysis-ready geometry record, frozen 10.750 m seven-station fuselage and 5.90 m quarter-chord tail-arm definition.

[7] G5_CGSM_ALPHA_M2_P2(1).csv, SM6.1 five-point local static-stability sweep.

[8] G5_CGSM_XREF390_CHECK.csv, SM6.2 moment-reference invariance audit.

[9] G5_SM63_WING_ONLY_ALPHA_M2_P2.csv, SM6.3A MainWing-only support run.

[10] Tyche R2-T1 SM6.3B Verification and SM6.4 CG-Envelope Closure, verified antecedent analysis record, 21 August 2026.