# Tyche R2-T1
Forward-CG Control and Gear Closure

Analysis Log and Conclusions through FG8.3

Folder: 05_Stability/08_Forward_CG_Control_Gear_Closure

Configuration: Tyche R2-T1 / G5 thin lifting-system production model

Status at issue: FG8.1 PASS | FG8.2 PASS | FG8.3 PASS | FG8.4 analytical gear-load closure pending

Purpose. This controlled log records the forward-CG longitudinal-control validation completed before the final analytical landing-gear load closure. It consolidates the executed VSPAERO cases, convergence checks, equations, derived rotation/trim results, model limitations, and release decisions so the findings can be transferred directly into the final design report.

# 1. Scope and Controlled Analysis Basis

Folder 08 was introduced after the SM6.4 loading analysis identified forward operational CG states that were more forward than the original nominal control boundary. The folder was intentionally limited to take-off rotation, landing trim/flare, and balked-landing flap-transition control. Powered free-wake VSPAERO was excluded because the VMC7.3 campaign had already established it as non-robust at the relevant low-speed, strongly blown condition.

| Item | Controlled value / decision |
| --- | --- |
| Aerodynamic model | G5 thin lifting-system production model; MainWing + HTail + VTail + LiftStrut_Pair |
| Thick-body model | Excluded; mixed thin/thick production use rejected in prior validation |
| Propulsion in VSPAERO | OFF for all FG8 cases |
| Take-off forward state | m = 1608.8 kg; xCG = FS 3.37374 m |
| Landing / go-around forward state | m = 1519.8 kg; xCG = FS 3.35170 m |
| Take-off Fowler surrogate | +12 deg, symmetric trailing-edge-down grouping |
| Landing Fowler surrogate | +35 deg, used only for longitudinal control screening |
| Go-around transition | 35 deg -> 20 deg -> 12 deg Fowler, quasi-steady |
| Elevator gains | +1 / -1 mirrored pair retained |
| Common numerical settings | VLM; 20 wake iterations; 16 wake nodes; fixed wake OFF; relaxation 1.0 |

## 1.1 Configuration-control notes

The first FG8.1 positive-elevator files were initially named with “M” despite containing positive GUI commands. The log therefore records the actual GUI command sign rather than relying on the filename token.

The aerodynamic response established that negative GUI elevator command produces the required nose-up pitching-moment direction in the current G5 control grouping. Final-report wording should therefore state physical control sense (nose-up / TE-up) and avoid relying only on the OpenVSP sign.

The early FG8.1 positive-command CSVs recorded rho = 1.225 kg/m^3 while the intended hot-high condition was rho = 1.079 kg/m^3. Because propulsion was OFF and Reynolds number was imposed explicitly, their nondimensional coefficients were retained. All dimensional rotation calculations were performed with rho = 1.079 kg/m^3, and the later negative-command runs used the corrected density.

# 2. FG8.0 Forward-CG Static Gear Baseline

Before aerodynamic rotation/flare testing, the tricycle-gear static reaction baseline was established using the controlled nose- and main-gear stations xN = 0.750 m and xM = 4.290 m.

R_N = W (x_M - x_CG) / (x_M - x_N)    (2.1)

R_M,total = W - R_N    (2.2)

| Condition | Mass (kg) | xCG (m) | Nose fraction | RN (kN) | Main total (kN) |
| --- | --- | --- | --- | --- | --- |
| Forward take-off | 1608.8 | 3.37374 | 25.88% | 4.084 | 11.693 |
| Forward landing | 1519.8 | 3.35170 | 26.51% | 3.950 | 10.954 |

Status. Static reaction baseline established. Dynamic/limit/ultimate landing-gear load closure remains FG8.4 and is intentionally not included as a completed result in this issue.

# 3. FG8.1 Hot-High Forward-CG Take-off Rotation

FG8.1 evaluated whether the actual forward take-off loading state can unload the nose wheel at the controlled hot-high rotation condition without credit for powered lift, propeller slipstream, or thrust-line pitching moment. This is deliberately conservative.

| Parameter | Value |
| --- | --- |
| Mass | 1608.8 kg |
| xCG / moment reference | FS 3.37374 m |
| Vinf | 22.16 m/s |
| rho | 1.079 kg/m^3 for dimensionalization |
| Mach | 0.0634 |
| Re(cbar) | 2.41E6 |
| Alpha / Beta | 0 deg / 0 deg |
| Fowler | +12 deg |
| Aileron / rudder | 0 deg / 0 deg |
| Propulsion | OFF |

## 3.1 Rotation moment balance

C_m,MG = C_m,CG + C_L (x_M - x_CG) / c_bar    (3.1)

M_aero,MG = q S c_bar C_m,MG    (3.2)

R_N = [ W (x_M - x_CG) - M_aero,MG ] / (x_M - x_N)    (3.3)

The rotation condition is satisfied when the calculated nose-wheel reaction reaches or crosses zero:

R_N <= 0    (3.4)

## 3.2 Executed elevator sweep and sign audit

| GUI elevator command | CL | Cm about CG | Interpretation / use |
| --- | --- | --- | --- |
| 0 deg | 0.84704 | +0.02659 | Neutral baseline |
| +5 deg | 0.91488 | -0.18134 | Opposite (nose-down moment) direction |
| +10 deg | 0.98171 | -0.38562 | Opposite direction |
| +15 deg | 1.04744 | -0.58513 | Opposite direction |
| +20 deg | 1.11211 | -0.78034 | Opposite direction |
| -15 deg | 0.64114 | +0.66859 | Nose-up authority; rotation crossing vicinity |
| -20 deg | 0.58037 | +0.86128 | Nose-up authority; converged positive margin |

## 3.3 Rotation result

| Case | Cm,MG | Maero,MG (kN m) | RN (kN) | Convergence disposition |
| --- | --- | --- | --- | --- |
| 0 deg | 0.4392 | 6.566 | +2.229 | Baseline |
| -15 deg | 0.9809 | 14.664 | -0.059 | Authority demonstrated; final-step convergence note |
| -20 deg | 1.1440 | 17.103 | -0.748 | PASS; longitudinal gates satisfied |

The minimum rotation demand is therefore approximately 15 deg in the demonstrated nose-up physical direction. Because the -15 deg case is noisier than the -20 deg case, the log deliberately avoids presenting a spurious high-precision minimum. The -20 deg point provides the clean positive-authority demonstration.

|delta_e,max,required| >= |delta_e,rot| / 0.90    (3.5)

Using an approximately 15 deg rotation demand, a 10% geometric control-travel reserve implies a minimum design requirement of approximately 17 deg in the nose-up elevator direction.

| FG8.1 convergence item | -15 deg | -20 deg | Gate / interpretation |
| --- | --- | --- | --- |
| CL 19->20 | 0.702% | 0.199% | <0.5%; -15 NOTE, -20 PASS |
| CLw 19->20 | 0.697% | 0.064% | <0.5%; -15 NOTE, -20 PASS |
| CDiw 19->20 | 0.775% | 0.102% | <0.5%; -15 NOTE, -20 PASS |
| ¦Delta Cm¦ 19->20 | 0.01432 | 0.00280 | <0.005; -15 NOTE, -20 PASS |

FG8.1 conclusion. PASS. Forward-CG hot-high take-off rotation authority is demonstrated without powered-lift, propwash, or thrust-line moment credit. Approximately 15 deg nose-up elevator is required; the converged 20 deg nose-up case provides a clear positive nose-wheel-unload margin. High-deflection lateral coefficients are not used as lateral-directional validation evidence.

# 4. FG8.2 Forward-CG Landing Trim and Flare Authority

FG8.2 first established that the +35 deg Fowler surrogate remained sufficiently stable for longitudinal control screening, then used neutral and -10 deg elevator alpha sweeps to bracket the actual forward-CG landing trim state.

## 4.1 +35 deg Fowler sanity check

| Quantity | Final value | 19->20 change | Disposition |
| --- | --- | --- | --- |
| CL | 1.59808 | 0.064% | PASS |
| CLw | 1.63859 | 0.267% | PASS |
| CDiw | 0.11955 | 0.513% | MARGINAL; not used as drag evidence |
| Cm | +0.06752 | 0.000264 absolute | PASS |
| CY / Cl / Cn | +0.00317 / +0.00206 / -0.00113 | - | Numerical-bias note; not used laterally |

Scope boundary. The +35 deg Fowler VLM state is accepted only as a longitudinal control-authority surrogate. It is not validation of the actual double-slotted Fowler/slat CLmax, landing drag polar, or certification-level stall behaviour.

## 4.2 Mass-specific forward landing reference state

C_L,req = W / (q S)    (4.1)

| Parameter | Value |
| --- | --- |
| Mass | 1519.8 kg |
| xCG | FS 3.35170 m |
| Vinf | 21.72 m/s |
| rho | 1.079 kg/m^3 |
| Mach | 0.0621 |
| Re(cbar) | 2.35E6 |
| q | 254.51 Pa |
| Required CL | 1.95198 |
| Fowler | +35 deg |
| Propulsion | OFF |

## 4.3 Executed trim sweeps

| Elevator | Alpha | CL | Cm |
| --- | --- | --- | --- |
| 0 deg | 0 deg | 1.59819 | +0.06743 |
| 0 deg | 4 deg | 1.96032 | -0.14137 |
| 0 deg | 8 deg | 2.30896 | -0.36888 |
| -10 deg | 0 deg | 1.46331 | +0.49134 |
| -10 deg | 4 deg | 1.82470 | +0.28110 |
| -10 deg | 8 deg | 2.17648 | +0.05205 |

## 4.4 Interpolated approach trim

At the required lift coefficient, the neutral-elevator sweep gives approximately alpha = 3.91 deg and Cm = -0.13656, whereas the -10 deg sweep gives approximately alpha = 5.45 deg and Cm = +0.19822. The two states therefore bracket the zero-moment trim point.

delta_e,trim ~= -4.08 deg (OpenVSP GUI command)    (4.2)

alpha_trim ~= 4.54 deg    (4.3)

The tested -10 deg state therefore retains approximately 5.9 deg of additional demonstrated nose-up elevator movement beyond steady trim. At CL = 1.95198, its additional positive pitching moment is approximately 2.85 kN m.

| FG8.2 result | Value |
| --- | --- |
| Required CL | 1.95198 |
| Neutral-elevator Cm at required CL | -0.13656 |
| -10 deg elevator Cm at required CL | +0.19822 |
| Estimated steady trim elevator | ~4.08 deg nose-up |
| Estimated trim alpha | ~4.54 deg |
| Tested additional elevator beyond trim | ~5.92 deg |
| Additional nose-up moment at -10 deg | ~2.85 kN m |

FG8.2 conclusion. PASS. The actual forward landing case trims within a small nose-up elevator demand, and the tested -10 deg state demonstrates substantial additional nose-up authority for conceptual flare screening. The result is a quasi-steady longitudinal control closure, not a dynamic flare or high-lift CLmax validation.

# 5. FG8.3 Balked-Landing / Go-Around Flap-Transition Screen

FG8.3 completed the planned VSPAERO portion of Folder 08. The test held the forward landing/go-around state fixed and changed only Fowler configuration to establish the pitching-moment trend during a quasi-steady 35 deg -> 20 deg -> 12 deg flap transition. Propulsion remained OFF in accordance with the powered-VSPAERO model-form closure.

## 5.1 Neutral-elevator transition data

| Alpha | CL @35 deg | Cm @35 deg | CL @20 deg | Cm @20 deg | CL @12 deg | Cm @12 deg |
| --- | --- | --- | --- | --- | --- | --- |
| 0 deg | 1.59819 | +0.06743 | 1.11818 | +0.03078 | 0.84699 | +0.01670 |
| 4 deg | 1.96032 | -0.14137 | 1.49415 | -0.16386 | 1.22843 | -0.16885 |
| 8 deg | 2.30896 | -0.36888 | 1.86278 | -0.37616 | 1.60411 | -0.37271 |

## 5.2 Convergence and physical trend

| Configuration | Alpha | CL 19->20 | ¦Delta Cm¦ 19->20 | CDiw 19->20 | Disposition |
| --- | --- | --- | --- | --- | --- |
| 20 deg | 0 deg | 0.053% | 0.00156 | 0.045% | PASS |
| 20 deg | 4 deg | 0.063% | 0.00091 | 0.635% | Longitudinal PASS; wake-drag note |
| 20 deg | 8 deg | 0.064% | 0.00058 | 0.267% | PASS |
| 12 deg | 0 deg | 0.042% | 0.00005 | 0.029% | PASS |
| 12 deg | 4 deg | 0.039% | 0.00018 | 0.051% | PASS |
| 12 deg | 8 deg | 0.021% | 0.00009 | 0.036% | PASS |

Across all alpha points the lift decreases monotonically as the flap is retracted, while the pitching-moment progression remains smooth. There is no configuration-induced pitch break or reversal. The single marginal wake-drag point is not used for performance evidence.

## 5.3 Approximate trim-continuity estimate

For the controlled log, the flap-transition pitch increment is evaluated at the established 35 deg landing trim attitude (alpha approximately 4.54 deg). A constant-required-lift elevator interpolation from FG8.2 gives an effective local trim sensitivity of approximately 0.0335 Cm per degree of nose-up elevator. This is more conservative than the preliminary same-alpha estimate and is used here for consistency.

Delta delta_e ~= - Delta C_m / (dC_m / d delta_e)    (5.1)

| Fowler state | Cm at alpha~4.54 deg | Delta Cm vs 35 deg | Approx. additional nose-up elevator | Approx. trim command |
| --- | --- | --- | --- | --- |
| 35 deg | -0.17184 | 0 | 0 | 4.08 deg |
| 20 deg | -0.19230 | -0.02046 | ~0.61 deg | ~4.69 deg |
| 12 deg | -0.19615 | -0.02431 | ~0.73 deg | ~4.81 deg |

Interpretation. These trim commands are quasi-steady configuration-transition estimates, not a powered six-degree-of-freedom go-around trajectory. The physical go-around simultaneously adds symmetric power and acceleration, reducing the required lift coefficient as speed increases. FG8.3 only demonstrates that flap retraction itself does not create an adverse/reversing pitch-control demand.

FG8.3 conclusion. PASS. The 35 deg -> 20 deg -> 12 deg transition shows monotonic lift reduction, smooth pitching-moment evolution, no pitch-control reversal, and only a small additional nose-up elevator increment relative to the already demonstrated authority. No additional VSPAERO runs are required for Folder 08.

# 6. Folder 08 Status Before FG8.4 Analytical Gear-Load Closure

| Test block | Question answered | Result | Release status |
| --- | --- | --- | --- |
| FG8.0 | Static forward-CG gear reactions established? | Yes | BASELINE COMPLETE; dynamic load closure pending |
| FG8.1 | Can the actual forward take-off case rotate at hot-high VR? | Yes | PASS / CLOSED |
| FG8.2 sanity | Is +35 deg Fowler usable for longitudinal control screening? | Yes, with drag/lateral limitations | PASS / NOTE |
| FG8.2 trim/flare | Can the actual forward landing case trim and retain nose-up authority? | Yes | PASS / CLOSED |
| FG8.3 | Does flap retraction introduce a pitch reversal or excessive trim demand? | No | PASS / CLOSED |
| FG8.4 | Do updated forward-CG gear limit/ultimate reactions close? | Not yet executed | NEXT / FINAL ANALYTICAL TASK |

## 6.1 Consolidated engineering conclusion

The forward-CG control problem introduced by the SM6.4 loading cloud is now substantially closed. The actual forward take-off case demonstrates nose-wheel unloading at the hot-high rotation condition using the unpowered +12 deg Fowler surrogate, while the actual forward landing case trims with only about 4.1 deg of nose-up elevator and retains substantial additional nose-up authority for flare. The quasi-steady 35 deg -> 20 deg -> 12 deg flap-transition screen shows smooth longitudinal behaviour and no pitch-control reversal. These results support retention of the current tail/elevator architecture without a late geometry redesign.

## 6.2 Model boundaries to carry into the final report

No powered free-wake VSPAERO result is used as production evidence in Folder 08. Symmetric thrust-line and powered-lift effects are intentionally not credited in FG8.1-FG8.3.

The Fowler geometries are OpenVSP control-surface surrogates. They are used for longitudinal control increments only, not to validate the actual slat/double-slot CLmax or landing drag polar.

High-deflection lateral coefficients in FG8.1/FG8.2 are not used for lateral-directional evidence; the validated G5 beta/rudder/aileron campaigns remain the controlled source for those derivatives.

FG8.3 is a quasi-steady configuration-transition screen. Propeller transient, flap-rate dynamics, ground effect, pilot response, and full six-degree-of-freedom go-around trajectory remain outside this competition-level closure.

Final landing-gear limit and ultimate reaction substantiation remains the only open Folder 08 item and will be completed analytically in FG8.4.

## 6.3 Recommended report-ready summary

Forward-CG control closure: At the actual forward take-off loading state (1608.8 kg, FS 3.37374 m), hot-high rotation authority was demonstrated without powered-lift or thrust-line moment credit, with approximately 15 deg nose-up elevator required and a converged 20 deg case providing positive nose-wheel-unload margin. At the actual forward landing/go-around state (1519.8 kg, FS 3.35170 m), the +35 deg Fowler control surrogate trimmed at approximately 4.1 deg nose-up elevator and retained substantial additional flare authority. A quasi-steady 35 deg -> 20 deg -> 12 deg flap-transition screen showed smooth lift and pitching-moment evolution with no pitch-control reversal and only a small additional elevator trim requirement. The current tail/elevator architecture therefore passes the conceptual forward-CG rotation, landing-trim/flare, and flap-transition control gates; landing-gear limit/ultimate reaction closure remains the final analytical item.

# Appendix A. Executed CSV Evidence Register

| Test | File(s) | Use in this log |
| --- | --- | --- |
| FG8.1 baseline / positive commands | FG8_1_HH_ROT_DE0.csv; FG8_1_HH_ROT_DE_M5.csv; ..._M10.csv; ..._M15.csv; ..._M20.csv | Elevator sign audit and opposite-direction response |
| FG8.1 nose-up authority | FG8_1_HH_ROT_DE_M15(1).csv; FG8_1_HH_ROT_DE_M20(1).csv | Main-wheel rotation closure |
| FG8.2 sanity | FG8_2_LDG35_SANITY_A0_DE0.csv | +35 deg Fowler numerical health |
| FG8.2 approach trim | FG8_2_LDG35_DE0_A0_4_8.csv | Neutral-elevator lift/pitch surface |
| FG8.2 flare authority | FG8_2_LDG35_DE_NEG10_A0_4_8.csv | Nose-up elevator trim bracket and reserve |
| FG8.3 transition 20 deg | FG8_3_GA20_DE0_A0_4_8.csv | Intermediate flap-transition state |
| FG8.3 transition 12 deg | FG8_3_GA12_DE0_A0_4_8.csv | Released take-off/go-around flap state |