# Tyche R2-T1 - 07_VMC_Control_Closure

Analysis Log, Test Evidence and Hybrid OEI Calculation Record

Tyche R2-T1 Design Study

23 August 2026

# Document control and executive disposition

This document consolidates the executed evidence, calculations, decisions and remaining release items from the 07_VMC_Control_Closure campaign. It covers the low-speed unpowered baseline (VMC7.1), the corrected +12 degree Fowler configuration (VMC7.2), every retained powered actuator-disk/free-wake diagnostic (VMC7.3 through VMC7.3F), and the deterministic hybrid one-engine-inoperative (OEI) equilibrium calculation (VMC7.4).

Controlled disposition. The low-speed unpowered and corrected-flap branches are accepted for their intended purposes. The coupled powered VSPAERO free-wake branch is closed as numerically non-robust at the heavily blown low-speed condition and is not used as a production source of OEI force or moment coefficients. Counter-rotation remains a favourable architecture sensitivity because it greatly improved equal-power left/right loading symmetry; no further numerical RPM-sign change is supported when one propeller geometry has already been physically reversed.

The VMC7.4 hybrid model gives the following conceptual-design control closure:

VMC,hybrid=38.73 kn EAS

for the governing VMC-A loading case, the windmilling failed-propeller bound CD,f=0.45, adverse live-propeller reaction torque, full provisional finite control travel, and no differential-thrust relief. The severe failed-propeller-drag sensitivity gives:

VMC,severe=39.09 kn EAS.

Both are below the internal sea-level design gate:

VMC,lim=0.95VR=39.96 kn EAS.

The result is a conditional analytical pass / release hold. At the gate, the windmilling case uses approximately 93.3% of provisional aileron travel and 93.7% of provisional rudder travel. A 10% simultaneous travel-reserve screen gives 40.73 kn and therefore misses the internal gate by 0.77 kn. The unresolved additional adverse OEI powered-lift rolling coefficient must satisfy approximately:

Cl,pw≤0.00250

at about 40 kn EAS unless control authority or the speed schedule is changed.

## Controlled status summary

| Campaign element | Controlled result | Disposition |
| --- | --- | --- |
| VMC7.1 low-speed unpowered | Stable 20-iteration solution at 20.58 m/s | PASS / CLOSED |
| VMC7.2 corrected Fowler +12 degrees | Symmetric configuration recovered; ΔCL=+0.4190 at alpha = 0 degrees | PASS / CLOSED for configuration verification |
| Initial powered CT=0.40 input | ηp=1.667>1 | REJECTED as physically inadmissible |
| Corrected disk screen | CT=0.1490, CP=0.07454, T=4.213 kN at 23.60 m/s | ACCEPTED AS BOUND |
| VMC7.3C through VMC7.3F | Persistent coefficient and residual oscillation | FAIL / powered branch closed |
| Counter-rotation sensitivity | Final Cl reduced about 81.6% relative to co-rotating RLX 0.5 | PHYSICS PASS / numerical fail |
| VMC7.4 full-travel windmill case | 38.73 kn EAS | PASS / THIN |
| VMC7.4 full-travel severe-drag case | 39.09 kn EAS | PASS / THIN |
| VMC7.4 90%-travel windmill case | 40.73 kn EAS | FAIL internal robustness target |
| Additional OEI powered-lift roll allowance | Cl,pw≤0.00250 | OPEN / RELEASE-CRITICAL |

# 1. Scope, conventions and acceptance philosophy

## 1.1 Aerodynamic coefficient mapping

The VSPAERO CSV history fields are mapped into the conventional aircraft-axis notation used in this record as follows. The signs of rolling and yawing moment are reversed from the raw CMxtot and CMztot fields to preserve the conventional Cl and Cn signs used by the G5 derivative set.

| Conventional quantity | VSPAERO history field | Mapping |
| --- | --- | --- |
| Lift coefficient, CL | CLtot | Direct |
| Wake lift coefficient, CL,w | CLwtot | Direct |
| Induced drag, CDi | CDi | Direct |
| Wake induced drag, CDi,w | CDiw | Direct |
| Side-force coefficient, CY | CStot | Direct |
| Rolling-moment coefficient, Cl | CMxtot | Cl=−CMxtot |
| Pitching-moment coefficient, Cm | CMytot | Direct |
| Yawing-moment coefficient, Cn | CMztot | Cn=−CMztot |

The final-five relative span used for lift and drag convergence checks is:

Span5,%X=100maxXN−4:N−minXN−4:NXN−4:N.

The campaign acceptance philosophy was:

| Quantity | Preferred numerical gate |
| --- | --- |
| CL, CL,w, CDi,w final-five relative span | less than 0.5% |
| Cm final-five absolute span | less than 0.005 |
| CY, Cl, Cn final-five absolute span | less than 5×10−4 preferred |
| Residuals | Sustained decay without late oscillation or sign cycling |

VMC7.1 and VMC7.2 were configuration-verification tests. VMC7.2 higher-alpha points were therefore judged primarily on the corrected flap implementation, symmetry and smooth aerodynamic trend; small wake-drag span exceedances were not promoted as production drag-polar evidence. VMC7.3 was intended to support powered OEI analysis and was held to the stricter production-coefficient standard.

## 1.2 Common reference geometry and low-speed states

| Parameter | Controlled value |
| --- | --- |
| Wing reference area, S | 30.0 m2 |
| Span, b | 16.4 m |
| Mean aerodynamic chord, c | 1.881 m |
| Primary VMC reference station | FS 3.70312 m for aerodynamic diagnostics |
| Propeller diameter, Dp | 2.00 m |
| Propeller lateral arm, yp | 1.60 m |
| Sea-level density used in CSV runs | 1.225 kg/m3 |
| VMC7.1-VMC7.2 speed | 20.58 m/s, Mach 0.0605, Re = 2.65×106 |
| VMC7.3 diagnostic speed | 23.60 m/s, Mach 0.0693, Re = 3.04×106 |

# 2. Complete campaign and file audit

The executed CSV evidence spans 22-23 August 2026. The first VMC7.2 and VMC7.3 files are intentionally retained because they record the discovery of the flap-grouping error. The (1) VMC7.1 file is byte-identical to the file without the suffix and is not a separate aerodynamic test.

| Test ID | Controlled source file | Purpose / configuration | Result and use |
| --- | --- | --- | --- |
| VMC7.1 | VMC7_1_LOWSPD_UNPWR_A0.csv | Alpha 0 degrees, beta 0 degrees, unpowered, low-speed baseline, 20 iterations | PASS; reference baseline |
| VMC7.1 duplicate | VMC7_1_LOWSPD_UNPWR_A0(1).csv | Same timestamp and byte content as VMC7.1 | Archive duplicate; no new test |
| VMC7.2 initial | VMC7_2_TOFLAP12_ALPHA_0_4_8.csv | Unpowered alpha 0/4/8 degrees; intended Fowler +12 degrees | REJECTED; asymmetric flap grouping produced spurious roll |
| VMC7.2 corrected | VMC7_2_TOFLAP12_ALPHA_0_4_8(1).csv | Corrected symmetric Fowler +12 degrees; alpha 0/4/8 degrees | PASS for flap implementation and low-speed trend |
| VMC7.3A initial | VMC7_3_SYM_TOFLAP12_ALPHA_0_4_8.csv | First powered symmetric alpha sweep | REJECTED; flap-grouping contamination and non-physical disk input basis |
| VMC7.3B corrected grouping | VMC7_3_SYM_TOFLAP12_ALPHA_0_4_8(1).csv | Corrected Fowler grouping, powered alpha 0/4/8 degrees, 20 iterations | Development evidence only; powered oscillation and old CT=0.40 basis |
| VMC7.3C | VMC7_3C_SYM_A0_40IT.csv | Corrected CT/CP, co-rotating, relaxation 1.0, 40 iterations | FAIL production convergence |
| VMC7.3D | VMC7_3D_SYM_A0_RLX05.csv | Same as VMC7.3C, relaxation 0.5 | Improved some spans; still FAIL |
| VMC7.3E | VMC7_3E_CR_SYM_A0_RLX05.csv | Counter-rotating, relaxation 0.5 | Physical symmetry improved; numerical FAIL |
| VMC7.3F | VMC7_3F_CR_SYM_A0_RLX025.csv | Counter-rotating, relaxation 0.25, 40 iterations | Final numerical test; FAIL and close powered branch |
| VMC7.4 | Deterministic hybrid calculation in this record | G5 derivatives + explicit OEI force/moment model + finite controls | CONDITIONAL ANALYTICAL PASS / RELEASE HOLD |

# 3. VMC7.1 - low-speed unpowered baseline

## 3.1 Test definition

The baseline isolated the low-speed airframe before adding Fowler deflection or actuator disks.

| Setting | Value |
| --- | --- |
| Alpha / beta | 0 / 0 degrees |
| Fowler, aileron, elevator, rudder | 0 / 0 / 0 / 0 degrees |
| Actuator disks | Off / zero loading |
| V∞ | 20.58 m/s |
| Mach / Reynolds number | 0.0605 / 2.65×106 |
| Density | 1.225 kg/m3 |
| Reference station | FS 3.70312 m |
| Wake iterations | 20 |

## 3.2 Final coefficients and numerical checks

| Quantity | Iteration-20 value | Final-five span | Gate | Result |
| --- | --- | --- | --- | --- |
| CL | 0.427911 | 0.0898% | less than 0.5% | PASS |
| CL,w | 0.428777 | 0.0656% | less than 0.5% | PASS |
| CDi,w | 0.006182 | 0.201% | less than 0.5% | PASS |
| Cm | +0.083816 | 0.000130 | less than 0.005 | PASS |
| CY | +0.001239 | 0.000204 | less than 5×10−4 | PASS |
| Cl | +0.000870 | 0.000061 | less than 5×10−4 | PASS |
| Cn | -0.000447 | 0.000015 | less than 5×10−4 | PASS |

The final residual values were log10L2=−1.8316 and log10Rmax=−0.2458. More importantly for this configuration-verification run, the aerodynamic coefficients were stable and the lateral coefficients remained small. VMC7.1 was therefore closed as the valid low-speed unpowered reference.

# 4. VMC7.2 - +12 degree Fowler implementation

## 4.1 Initial flap-grouping failure

The first VMC7.2 sweep showed a large rolling coefficient in a nominally symmetric, unpowered condition. At alpha = 0 degrees the initial file gave Cl=−0.063141, compared with Cl=+0.001371 after correcting the flap grouping. The correction also reduced the side-force and yawing-moment magnitudes.

| Alpha = 0 degrees quantity | Initial file | Corrected file | Change |
| --- | --- | --- | --- |
| CL | 0.427900 | 0.846943 | Fowler lift increment recovered |
| CY | +0.011656 | +0.001657 | 85.8% reduction in magnitude |
| Cl | -0.063141 | +0.001371 | 97.8% reduction in magnitude |
| Cn | -0.002730 | -0.000641 | 76.5% reduction in magnitude |
| Cm | +0.078101 | +0.174848 | Corrected flap load/moment state |

The initial file is retained as a diagnostic failure, not as aerodynamic evidence. It proved that the observed large roll was created by control-surface grouping rather than by the clean aircraft.

## 4.2 Corrected unpowered Fowler sweep

| Alpha (degrees) | CL | CL,w | CDi,w | Cm |
| --- | --- | --- | --- | --- |
| 0 | 0.846943 | 0.851300 | 0.027738 | +0.174848 |
| 4 | 1.228337 | 1.234087 | 0.052715 | +0.061235 |
| 8 | 1.604105 | 1.614415 | 0.087709 | -0.072316 |

| Alpha (degrees) | CY | Cl | Cn | Lateral-state interpretation |
| --- | --- | --- | --- | --- |
| 0 | +0.001657 | +0.001371 | -0.000641 | Small corrected residual |
| 4 | +0.000492 | +0.000394 | -0.000175 | Approaching zero |
| 8 | -0.000689 | -0.000443 | +0.000189 | Gentle sign reversal |

| Alpha (degrees) | CL span | CL,w span | CDi,w span | Cm absolute span | Assessment |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.070% | 0.069% | 0.105% | 0.000265 | PASS |
| 4 | 0.639% | 0.612% | 0.750% | 0.001931 | Minor wake-span exceedance; trend retained |
| 8 | 0.123% | 0.175% | 0.805% | 0.000439 | Minor wake-drag span exceedance; trend retained |

The low-speed Fowler lift increment at alpha = 0 degrees is:

ΔCL,F12=CL,F12−CL,clean=0.846943−0.427911=0.419032.

The three-point lift-curve slope over 0-8 degrees is approximately:

CLα≈1.604105−0.8469438π/180=5.423 rad−1.

The corrected lateral coefficients remain small and reverse gently about zero across the alpha sweep. VMC7.2 was therefore closed for its intended purpose: establishing a symmetric +12 degree Fowler surrogate and a credible unpowered low-speed loading trend. It was not used to create a high-fidelity separated-flow drag polar.

# 5. VMC7.3 - powered actuator-disk/free-wake campaign

## 5.1 Physical screening of actuator-disk inputs

For propeller rotational speed n in revolutions per second, the coefficient definitions used in the campaign screen are:

CT=Tρn2D4,  CP=Pρn3D5.

The advance ratio and propulsive efficiency are:

J=VnD,  ηp=JCTCP=TVP.

At 2279 RPM, D=2.00 m, V=23.60 m/s and ρ=1.225 kg/m3:

n=227960=37.983 s−1,  J=0.31066.

The original CT=0.40 with CP=0.07454 implies:

T=11.311 kN,  P=160.12 kW,  ηp=1.667.

Because ηp>1, the input is physically inadmissible and all old CT=0.40 results are rejected for quantitative use.

The corrected screen uses:

CT=0.1490,  CP=0.07454,

which gives:

T=4.213 kN,  P=160.12 kW,  ηp=0.6210.

This point is accepted only as a conservative idealized thrust/power bound, not as a validated production propeller map.

## 5.2 Early powered alpha sweeps

| Source | Alpha = 0 degrees final state | Diagnostic conclusion |
| --- | --- | --- |
| Initial VMC7_3...csv | CL=0.88396, CY=+0.00725, Cl=−0.09143, Cm=+0.49504 | Dominated by flap-grouping error; reject |
| Corrected-grouping VMC7_3...(1).csv | CL=1.26838, CY=−0.00704, Cl=+0.01540, Cm=+0.64453 | Flap asymmetry removed, but powered iteration remains oscillatory and old CT basis is invalid |

These runs separated the control-grouping error from the powered-flow problem. They are retained as troubleshooting evidence only.

## 5.3 Controlled 40-iteration diagnostic settings

| Setting | VMC7.3C | VMC7.3D | VMC7.3E | VMC7.3F |
| --- | --- | --- | --- | --- |
| Alpha / beta | 0 / 0 degrees | 0 / 0 degrees | 0 / 0 degrees | 0 / 0 degrees |
| Fowler / other controls | +12 / 0 degrees | +12 / 0 degrees | +12 / 0 degrees | +12 / 0 degrees |
| V∞ | 23.60 m/s | 23.60 m/s | 23.60 m/s | 23.60 m/s |
| RPM magnitude | 2279 | 2279 | 2279 | 2279 |
| CT/CP | 0.1490 / 0.07454 | Same | Same | Same |
| Rotation architecture | Co-rotating | Co-rotating | Counter-rotating | Counter-rotating |
| Wake nodes / iterations | 16 / 40 | 16 / 40 | 16 / 40 | 16 / 40 |
| Fixed wake | Off | Off | Off | Off |
| Relaxation | 1.0 | 0.5 | 0.5 | 0.25 |

## 5.4 Final coefficients

| Test | CL | CL,w | CDi,w | Cm |
| --- | --- | --- | --- | --- |
| VMC7.3C | 1.27814 | 1.41405 | 0.031024 | +0.62514 |
| VMC7.3D | 1.28147 | 1.41374 | 0.032805 | +0.61839 |
| VMC7.3E | 1.29625 | 1.45414 | 0.043366 | +0.66299 |
| VMC7.3F | 1.34860 | 1.48845 | 0.045933 | +0.49502 |

| Test | CY | Cl | Cn | Lateral interpretation |
| --- | --- | --- | --- | --- |
| VMC7.3C | 0.010801 | 0.016040 | 0.005146 | Co-rotating asymmetry |
| VMC7.3D | 0.007525 | 0.016238 | 0.003925 | Asymmetry persists |
| VMC7.3E | 0.001742 | 0.002991 | 0.000582 | Counter-rotation improvement |
| VMC7.3F | 0.001706 | 0.001993 | 0.000592 | Small instant, still oscillatory |

Final values alone are not accepted as converged results. They are shown to document the direction of the architecture and relaxation sensitivities.

## 5.5 Final-five convergence audit

| Test | CL span | CL,w span | CDi,w span | Cm span |
| --- | --- | --- | --- | --- |
| VMC7.3C | 3.26% | 2.80% | 12.48% | 0.12857 |
| VMC7.3D | 1.77% | 0.90% | 4.42% | 0.07542 |
| VMC7.3E | 4.11% | 2.11% | 3.82% | 0.17326 |
| VMC7.3F | 3.91% | 2.72% | 8.24% | 0.15738 |

| Test | CY span | Cl span | Cn span | Lateral gate result |
| --- | --- | --- | --- | --- |
| VMC7.3C | 0.022017 | 0.001835 | 0.008318 | FAIL |
| VMC7.3D | 0.004114 | 0.002461 | 0.001707 | FAIL |
| VMC7.3E | 0.005732 | 0.000890 | 0.002242 | FAIL |
| VMC7.3F | 0.005705 | 0.000882 | 0.002167 | FAIL |

Every controlled powered case fails multiple gates. Reducing relaxation from 1.0 to 0.5 improved some lift and wake spans but did not establish convergence. Reducing it again to 0.25 did not produce monotonic residual decay and worsened several measures.

## 5.6 Residual audit

| Test | Final log10L2 | Final log10Rmax | Observation |
| --- | --- | --- | --- |
| VMC7.3C | -0.2859 | +1.0153 | Oscillatory, no sustained decay |
| VMC7.3D | -0.3463 | +1.3910 | Relaxation did not close residuals |
| VMC7.3E | -0.0873 | +1.3677 | Counter-rotation fixed symmetry, not iteration stability |
| VMC7.3F | -0.4550 | +1.2570 | Final numerical test still oscillatory |

The final five iterations of VMC7.3F show the failure directly.

| Iteration | CL | CL,w | CDi,w | Cm |
| --- | --- | --- | --- | --- |
| 36 | 1.29886 | 1.45337 | 0.042352 | 0.65096 |
| 37 | 1.29735 | 1.44869 | 0.042604 | 0.65240 |
| 38 | 1.30070 | 1.45718 | 0.042745 | 0.64391 |
| 39 | 1.30631 | 1.45525 | 0.043748 | 0.62739 |
| 40 | 1.34860 | 1.48845 | 0.045933 | 0.49502 |

| Iteration | CY | Cl | Cn | log10L2 | log10Rmax |
| --- | --- | --- | --- | --- | --- |
| 36 | +0.002278 | +0.002351 | -0.000877 | -0.206 | 1.421 |
| 37 | +0.002343 | +0.002875 | -0.000964 | -0.354 | 1.142 |
| 38 | -0.003362 | +0.002782 | +0.001203 | -0.115 | 1.541 |
| 39 | -0.002219 | +0.002669 | +0.000822 | -0.315 | 1.552 |
| 40 | +0.001706 | +0.001993 | -0.000592 | -0.455 | 1.257 |

From iteration 39 to 40:

1.34860−1.306311.34860=+3.14%,  1.48845−1.455251.48845=+2.23%,

0.045933−0.0437480.045933=+4.76%,  ΔCm=0.49502−0.62739=0.13237.

The lateral coefficients also change sign rather than settling toward zero. VMC7.3F is therefore a numerical failure even though its iteration-40 lateral coefficients happen to be small.

## 5.7 Counter-rotation and RPM-sign conclusion

At the same relaxation setting, changing from co-rotation (VMC7.3D) to counter-rotation (VMC7.3E) changed the final lateral magnitudes as follows:

| Quantity | Co-rotating RLX 0.5 | Counter-rotating RLX 0.5 | Magnitude reduction |
| --- | --- | --- | --- |
| CY | 0.007525 | 0.001742 | 76.8% |
| Cl | 0.016238 | 0.002991 | 81.6% |
| Cn | 0.003925 | 0.000582 | 85.2% |

These ratios are descriptive architecture sensitivities, not converged production coefficients. Together with the substantially more symmetric wing-load distribution, they support opposite physical rotation directions for the two propellers.

If counter-rotation was created by reversing one propeller geometry with the OpenVSP Rev control while both disk-tab RPM magnitudes remain +2279, the numeric RPM signs should remain unchanged. Negating one RPM numerically on top of the geometric reversal would risk double-reversing the intended rotation definition. The frozen conclusion is:

Opposite physical rotation produced substantially better equal-power propwash symmetry. Numerical RPM sign alone is not used as the architecture definition, and no additional RPM-sign change is supported by the evidence.

Supplier confirmation of an opposite-rotation hardware pair remains open.

## 5.8 VMC7.3 closure decision

Further attempts with 60-80 iterations, relaxation 0.1, arbitrary wake-node changes or unvalidated CT reductions were not justified. After bounded diagnostics across rotation sense and relaxation, continued tuning would select a numerical setting rather than validate the physical model. The controlled decision is:

Coupled powered VSPAERO branch closed; not production-valid at the VMC condition.

# 6. VMC7.4 - hybrid OEI force-and-moment equilibrium

## 6.1 Method statement

VMC7.4 replaces the rejected coupled powered solution with a deterministic steady OEI balance. It uses only converged G5 airframe derivatives and introduces propulsion asymmetry as explicit external forces and moments. No differential-thrust relief is credited. The left- and right-engine-loss cases are mirror images; the adverse propeller-torque sense is retained.

## 6.2 Actual loading cases

| Case | Mass | CG station | Role |
| --- | --- | --- | --- |
| VMC-A | 1894.8 kg | FS 3.70312 m | Most-aft actual departure case; primary |
| VMC-B | 2000.0 kg | FS 3.65023 m | Most-aft MTOM case |
| VMC-C | 1900.0 kg | FS 3.63687 m | Most-aft planned landing-mass case |

The artificial VMC-U aft sensitivity at FS 3.83778 m remains an envelope audit item but is not used as the headline actual-loading result.

## 6.3 Converged G5 derivative block

| Derivative | G5 value (rad-1) | Applied VMC7.4 value |
| --- | --- | --- |
| CYβ | -0.42565 | -0.42565 |
| Clβ | -0.071802 | -0.071802 |
| Cnβ | +0.13247 | CG-transferred |
| CYδa | +0.006868 | 0.75 times derivative |
| Clδa | -0.081393 | 0.75 times derivative |
| Cnδa | +0.0049793 | CG-transferred, then times 0.75 |
| CYδr | -0.29803 | 0.85 times derivative |
| Clδr | -0.02735 | 0.85 times derivative |
| Cnδr | +0.11056 | CG-transferred, then times 0.85 |

The near-stall control-effectiveness debits are:

ηa=0.75,  ηr=0.85.

For example:

Clδa,eff=0.75−0.081393=−0.061045,

Cnδr,effxref=0.850.11056=0.093976.

These debits are conservative conceptual-design allowances, not claims that the local high-lift control flow has been experimentally validated.

## 6.4 Transfer of yaw derivatives to each CG

The G5 coefficients were generated about xref=3.750 m. A lateral force creates a yaw-moment change when the moment reference moves longitudinally:

CnχxCG=Cnχxref+xCG−xrefbCYχ,  χ∈{β,δa,δr}.

For VMC-A sideslip stability:

Cnβ,A=0.13247+3.70312−3.75016.4−0.42565=0.133687.

After applying the control debits:

| Case | Cnβ | Cnδa,eff | Cnδr,eff |
| --- | --- | --- | --- |
| VMC-A | 0.133687 | 0.003720 | 0.094700 |
| VMC-B | 0.135059 | 0.003703 | 0.095517 |
| VMC-C | 0.135406 | 0.003699 | 0.095723 |

VMC-A consequently has the weakest directional authority of the actual cases and is expected to govern.

## 6.5 Live-engine thrust bound

The surviving-side power screen is:

Plive=160.13 kW.

With propeller disk area:

Ap=πDp24=3.1416 m2,

the induced velocity vi is obtained at each speed from the ideal actuator-disk relation:

Plive=2ρApviV+vi2,

followed by:

Tlive=2ρApviV+vi.

The local hot-day density used for the thrust model is 1.168 kg/m3; dynamic pressure for the control coefficients is EAS-referenced. The ideal disk deliberately gives a high-thrust directional-control bound. At the governing VMC-A windmilling point the live-engine thrust is approximately 4.37 kN.

## 6.6 Failed-propeller drag envelope

Failed-propeller drag is represented by:

Df=qApCD,f,  q=12ρ0VE2.

| Failed-propeller state | CD,f | Use |
| --- | --- | --- |
| Feathered / low drag | 0.08 | Favourable bound |
| Intermediate / stopped | 0.25 | Sensitivity |
| Windmilling control case | 0.45 | Primary VMC7.4 case |
| Severe windmilling bound | 0.60 | Adverse sensitivity |

At the 39.96 kn EAS gate, q=258.84 Pa and the corresponding failed-propeller drags are 65.1, 203.3, 365.9 and 487.9 N. These coefficients are inherited bounding sensitivities, not R2 propeller-test data.

## 6.7 Propeller reaction torque

At 2279 RPM:

ω=2π227960=238.66 rad s−1,

Qp=Pliveω=671.0 N m.

The rolling coefficient is:

Cl,Q=sQQpqSb,

where sQ is selected so that the reaction torque adds to the adverse OEI roll requirement. At 39.96 kn EAS:

Cl,Q=0.00527.

## 6.8 Simultaneous equilibrium equations

The unknowns at each candidate speed and permitted bank angle are sideslip, aileron and rudder deflection:

x=βδaδrT.

Side-force equilibrium is:

CYββ+ηaCYδaδa+ηrCYδrδr+WsinϕqS=0.

Roll equilibrium is:

Clββ+ηaClδaδa+ηrClδrδr+Cl,Q+Cl,pw=0.

The OEI yaw disturbance is:

Cn,OEI=Tlive+DfypqSb,

and yaw equilibrium is:

Cnβ,CGβ+Cnδa,CGδa+Cnδr,CGδr+Cn,OEI=0.

The combined matrix equation is:

CYβηaCYδaηrCYδrClβηaClδaηrClδrCnβ,CGCnδa,CGCnδr,CGβδaδr=−Wsinϕ/qSCl,Q+Cl,pwCn,OEI.

## 6.9 Finite limits and feasibility definition

The provisional physical stops are:

δr≤35∘,  δa≤22∘.

The additional robustness screen retains 10% travel reserve:

δr≤31.5∘,  δa≤19.8∘.

The other limits are:

β≤5∘,  ϕ≤5∘.

For a given speed, define the minimum normalized utilization over permitted bank angle as:

UV=minϕ≤5∘maxβ5∘,δaδa,lim,δrδr,lim.

The hybrid minimum control speed is the lowest speed with a feasible equilibrium:

VMC=min{V:UV≤1}.

## 6.10 Computational sequence

- Sweep EAS from 35 to 44 kn, then refine around the feasibility crossing.

- Calculate EAS-referenced dynamic pressure.

- Convert EAS to local TAS for the ideal live-thrust calculation.

- Solve the momentum relation for induced velocity and live thrust.

- Calculate failed-propeller drag, OEI yaw coefficient and reaction-torque rolling coefficient.

- Sweep permitted bank toward the live engine.

- Solve the 3 by 3 equilibrium for β, δa and δr.

- Reject solutions exceeding sideslip, bank or finite control limits.

- Mirror the disturbance for the opposite failed side and retain the adverse torque sense.

- Take the largest VMC across VMC-A/B/C and the bounded failed-propeller states.

# 7. VMC7.4 results

## 7.1 Full provisional physical travel

| CG case | CD,f=0.08 | 0.25 | 0.45 windmill | 0.60 severe |
| --- | --- | --- | --- | --- |
| VMC-A | 37.83 kn | 38.22 kn | 38.73 kn | 39.09 kn |
| VMC-B | 37.69 kn | 38.08 kn | 38.56 kn | 38.96 kn |
| VMC-C | 37.67 kn | 38.03 kn | 38.54 kn | 38.95 kn |

VMC-A governs every failed-propeller-drag state. Relative to the internal 39.96 kn gate, the primary windmilling result retains 1.23 kn and the severe-drag result retains 0.87 kn.

## 7.2 Governing VMC-A windmilling equilibrium

At VMC=38.729 kn EAS and CD,f=0.45:

| Variable | Equilibrium magnitude |
| --- | --- |
| Live-engine thrust | 4.37 kN |
| Failed-propeller drag | 0.344 kN |
| Reaction-torque coefficient, Cl,Q | 0.00561 |
| Bank toward live engine, ϕ | 4.00 degrees |
| Sideslip, β | 2.95 degrees |
| Aileron, δa | 22.00 degrees |
| Rudder, δr | 34.84 degrees |

Aileron reaches its finite stop before the 5 degree bank or 5 degree sideslip limits. Rudder is also nearly saturated. The governing result is therefore a simultaneous roll-yaw VMC rather than a rudder-only yaw calculation.

## 7.3 Ten-percent simultaneous travel reserve

| CG case | CD,f=0.08 | 0.25 | 0.45 windmill | 0.60 severe |
| --- | --- | --- | --- | --- |
| VMC-A | 39.62 kn | 40.14 kn | 40.73 kn | 41.15 kn |
| VMC-B | 39.52 kn | 39.95 kn | 40.53 kn | 40.99 kn |
| VMC-C | 39.47 kn | 39.95 kn | 40.52 kn | 40.99 kn |

The windmilling 90%-travel result exceeds the 39.96 kn internal gate by 0.77 kn. This is not a failure of physical minimum control speed; it shows that the present conservative model does not retain a full 10% simultaneous control-travel reserve at the gate.

## 7.4 Control use at the 39.96 kn gate

For VMC-A, CD,f=0.45 and adverse shaft torque:

δa≈20.52∘=93.3% of 22 degrees,

δr≈32.78∘=93.7% of 35 degrees.

Remaining geometric travel is approximately 6.7% aileron and 6.3% rudder. Under the severe CD,f=0.60 case, both controls rise to roughly 95-96% travel. The appropriate engineering description is pass at physical stops, but with thin simultaneous control reserve.

## 7.5 Unresolved powered-lift rolling moment

The rejected powered solver cannot supply a trustworthy one-sided powered-lift rolling coefficient. VMC7.4 therefore sets an allowable release threshold instead of manufacturing a value from the total powered lift increment.

At 39.96 kn EAS, the maximum tolerable total adverse roll disturbance is approximately:

Cl,dist,max≈0.00777.

Reaction torque already consumes:

Cl,Q≈0.00527.

The remaining allowance is:

Cl,pwmax≈0.00777−0.00527=0.00250.

With qSb=127349.8 N m at the gate, this corresponds to:

Lpw,max=qSbCl,pw≈318 N m.

If expressed only as an equivalent one-side lift decrement at yp=1.60 m:

ΔCL,side,max=Cl,pwbyp=0.0025016.41.60=0.0256.

This is a release requirement, not a prediction of the actual asymmetric lift.

| Added adverse Cl,pw | Calculated VMC-A, CD,f=0.45 |
| --- | --- |
| 0 | 38.73 kn |
| 0.001 | 39.20 kn |
| 0.002 | 39.70 kn |
| 0.0025 | approximately 39.96 kn |
| 0.003 | 40.21 kn |
| 0.005 | 41.29 kn |
| 0.010 | 44.49 kn |

# 8. Decision ledger and release items

| Evidence item | Result | Disposition / required action |
| --- | --- | --- |
| Low-speed clean-airframe numerical stability | Coefficients stable at 20.58 m/s | PASS |
| Fowler grouping | Initial asymmetry identified and corrected | PASS / preserve rejected file in audit trail |
| Corrected +12 degree Fowler state | Symmetric low-speed trend established | PASS for VMC configuration basis |
| Original disk CT=0.40 | ηp=1.667 | REJECT |
| Corrected CT/CP bound | 4.213 kN and 160.12 kW at 23.60 m/s | PASS AS BOUND |
| Co-rotating powered wake | Large lateral asymmetry plus oscillation | REJECT as production evidence |
| Counter-rotating powered wake | Large symmetry improvement | FAVOURABLE architecture sensitivity |
| Relaxation 1.0 / 0.5 / 0.25 | No robust convergence after 40 iterations | CLOSE powered branch |
| RPM-sign question | Geometric reversal already establishes physical counter-rotation | Do not negate one RPM again; supplier confirmation open |
| G5 beta, rudder and aileron derivatives | Converged and linear in the validated range | PASS |
| Actual VMC-A/B/C CG transformation | Completed | PASS |
| Ideal live-thrust and reaction-torque bounds | Included | PASS AS BOUNDS |
| Failed-propeller CD=0.08 to 0.60 | Sensitivity envelope only | HOLD - obtain supplier forward/feather/windmill map |
| R2 rudder/aileron stops | plus or minus 35 / 22 degrees provisionally inherited | HOLD - freeze mechanical geometry |
| VMC-A windmill, full travel | 38.73 kn | PASS / THIN |
| VMC-A severe drag, full travel | 39.09 kn | PASS / THIN |
| VMC-A windmill, 90% travel | 40.73 kn | FAIL internal robustness target |
| Additional adverse powered-lift roll | Must remain at or below about 0.00250 at the gate | OPEN / RELEASE-CRITICAL |

## 8.1 Required evidence before final VMC release

- Freeze the R2 mechanical rudder and aileron stops and demonstrate control-system load capability at the required simultaneous deflections.

- Obtain or defensibly bound the actual propeller forward-thrust, feathered, stopped and windmilling drag maps over the VMC advance-ratio range.

- Establish the OEI asymmetric powered-lift rolling coefficient using a higher-fidelity method or test and demonstrate Cl,pw≤0.00250 at approximately 40 kn EAS, or revise control authority / operating speeds.

- Confirm that the intended supplier architecture supports opposite physical propeller rotation without relying on an ambiguous numerical RPM sign convention.

- Repeat the final left- and right-side loss calculations with frozen propeller maps, control stops, masses and CGs.

# 9. Controlled conclusion

VMC7.1 proved that the low-speed unpowered numerical model is stable. VMC7.2 identified and corrected the Fowler grouping error and established a symmetric +12 degree low-speed state. VMC7.3 then separated two different powered-flow issues: co-rotation created a real equal-power lateral asymmetry, while counter-rotation largely removed that asymmetry; however, neither counter-rotation nor relaxation reductions from 1.0 to 0.5 and 0.25 produced a converged coupled actuator-disk/free-wake solution. The powered branch is therefore closed as a documented model-form/numerical limitation.

VMC7.4 provides the defensible conceptual-design replacement. Using converged G5 derivatives, actual VMC-A/B/C CGs, explicit ideal live-thrust and failed-propeller drag bounds, adverse reaction torque, finite controls, 5 degree bank and sideslip limits, and no differential-thrust credit, VMC-A governs. The windmilling full-travel result is 38.73 kn EAS and the severe-drag result is 39.09 kn EAS, both below the 39.96 kn internal gate. Because the 90%-travel windmilling result is 40.73 kn and only Cl,pw≈0.00250 additional adverse rolling coefficient remains at the gate, the campaign closes as:

CONDITIONAL ANALYTICAL PASS / RELEASE HOLD

The current report may state the hybrid values only with the failed-propeller-drag, control-stop, reaction-torque and unresolved powered-lift-roll qualifications preserved.

# References and controlled evidence

- Royal Aeronautical Society, International Light Aircraft Design Competition Rules 2025/26.

- Tyche R2-T1, SM6.3B Verification and SM6.4 CG-Envelope Closure - Pre-VMC Handoff, controlled VMC-A/B/C loading cases.

- VMC7_1_LOWSPD_UNPWR_A0.csv, executed 22 August 2026.

- VMC7_2_TOFLAP12_ALPHA_0_4_8.csv and corrected VMC7_2_TOFLAP12_ALPHA_0_4_8(1).csv, executed 22 August 2026.

- VMC7_3_SYM_TOFLAP12_ALPHA_0_4_8.csv and corrected-grouping VMC7_3_SYM_TOFLAP12_ALPHA_0_4_8(1).csv, executed 22 August 2026.

- VMC7_3C_SYM_A0_40IT.csv, executed 22 August 2026.

- VMC7_3D_SYM_A0_RLX05.csv, executed 22 August 2026.

- VMC7_3E_CR_SYM_A0_RLX05.csv, executed 22 August 2026.

- VMC7_3F_CR_SYM_A0_RLX025.csv, executed 23 August 2026.

- Tyche R2-T1 G5 converged sideslip, rudder and aileron derivative campaign records.